Habitats Directive for the water body in question irrespective of the proposed abstraction. The feasibility, methodology and monitoring requirements of addressing these cumulative impacts would need to be considered carefully, in consultation with NPWS.

NPWS is currently reviewing notifiable activities for different habitats and species, considering types of exempted developments within and potentially impacting on conservation sites, and preparing new guidance documentation. It is understood that new Regulations will also be forthcoming.

6.   Documenting the Stage 1 decision-making process in the event of a finding of likely significant effects on N2K sites.

Following completion of the review of options and issues discussed above, the findings of the Stage 1 screening assessment should be documented. This documentation can then be used as a basis for proceeding with further consideration of the selected project option, and in submissions to and consultations with the relevant agencies as the project is developed.

### Stage 2 Assessment.

Consultation with NPWS via the Development Applications Unit of the DoEHLG is recommended at an early stage, so that recommendations on detailed ecological and hydrological studies required can be included in the Stage 2 studies. Baseline studies of habitats, flora and fauna are likely to be subject to seasonal factors, and in some cases may need to take place over more than one year; for example populations of Vertigo species at individual sites can show very large natural fluctuations between years, and it may be necessary to take these into account in order to provide a sound basis for the assessment of potential impacts. It may also be useful to consult with Non-Governmental Organisations, and to seek local information about the area. Consultation should include reference to possible mitigation measures, since the plan proponent’s and the assessment team’s views about effective mitigation may vary from those of the Competent Authority and other stakeholders. If the Article 6 assessment forms part of an Environmental Impact Statement or Strategic Environmental Statement, it should be clearly identified in the document, and the entire project team should be made aware of the issues arising in relation to Natura 2000 sites.

Construction phase impacts may include the site footprint, the construction compound and site access, trenching, temporary or permanent modifications to hydrology, release of silt and other pollutants to watercourses. Operational phase impacts may include abstractions, discharges (point and diffuse), maintenance works, changes in hydrology, channel obstruction, changes in land use and associated pressures, and associated impacts on habitats, flora and fauna. The sensitivity of impacted habitats and species to impacts arising from the proposal, and to cumulative changes in water status should be clearly established for the relevant individual features. It may be necessary to consider the impacts of the proposal and the cumulative impacts on habitats, flora and fauna in the context of their occurrence within the affected Natura 2000 site(s), and also in the context of their regional and national distribution, so that the significance of those impacts can be fully assessed. Regional or site-specific variation in the typology of a habitat may also be relevant; an objective of the Habitats Directive is to maintain the full range of expression of Annex 1 listed habitats as a biodiversity measure.

Mitigation measures proposed should be backed up with sufficient evidence to indicate that they are workable. For example, if seasonal restrictions on construction activity are to be applied to avoid disturbance impacts to wintering waterfowl in an SPA, site investigation of soils/subsoils or depth to bedrock may be needed, to demonstrate that the construction activity can be completed within the time frame indicated. Mitigation measures may also need to be backed up by contingency measures for low flow conditions. For example, an abstraction of specified volume may be ecologically sustainable under high or average flow or recharge conditions, but not at low flow or recharge, and it may be necessary to provide supplementary water storage elsewhere for use at these times. Seasonal and life cycle variations in the sensitivities of impacted habitats, flora and fauna will also have to be taken into account.

Discharges of specified volume and concentration are more problematic because assimilative capacity will vary seasonally, and not necessarily in relation to the annualised average periodicity of high, average or low flows in receiving watercourses. In situations where the assimilative capacity of receiving waters is known to be low or marginal because of a variety of factors including downstream habitat and species sensitivity and existing diffuse pressures, waste water treatment plants (small and large scale) may need to incorporate additional treatments to improve effluent quality prior to discharge. These could include reed-bed treatment systems, and/or second and third generation biofuel technologies, for example. It should be noted that some lakes where water dependent Annex 2 listed species occur, and some coastal lagoons, have been assessed as being at unfavourable conservation status with small scale and larger WWTP noted as a contributory factor.

For plans and projects where mitigation measures have been incorporated into the project design, this should be clearly indicated in the Article 6 assessment, and in all relevant sections of the EIS documentation. For example, a road construction project in a karst area, or other area where ground water dependent habitats are present, may be designed to run at or above grade in key sections in order to avoid disruption of existing ground water flows to ground water dependent habitats and species. In these kinds of areas, excavation and trenching work would have a potential to result in adverse impacts to water dependent protected areas, habitats and species.

Where mitigation measures are applied to avoid, reduce or remedy impacts on protected areas and other water dependent habitats and species, it will be necessary to monitor the effectiveness of the measures applied over an appropriate period of time, so that the measures can be adjusted if necessary, and also to inform future decision making by Competent Authorities. It will also be necessary to establish protocols, and if necessary binding legal requirements, to ensure that mitigation measures continue to be implemented as time passes. This kind of situation could arise if one or more sluices were to be provided to regulate water levels as part of a mitigation measure for a water dependent habitat or species, for example. It would be necessary to ensure that sluices were maintained and operated in accordance with the ecological requirements of the habitat and/or species, taking seasonal factors into account, and also being responsive to long term trends and additional management measures that may be required from time to time (e.g. removal of accumulated leaf litter or developing scrub cover, review of grazing regimes etc.). A mechanism to ensure that sluices are not tampered with by third parties may also be required.

Monitoring and maintenance of mitigation measures is an issue that can be considered in a Stage 2 assessment carried out by, or on behalf of a project proponent. However, it is a function of the Competent Authorities to assess whether the mitigation and monitoring measures proposed in an assessment are appropriate or adequate, and to apply conditions to any grant of permission or licence accordingly.

### Stage 3 Assessment.

Stage 3 of an Article 6 assessment is undertaken by the Competent National Authorities, in consultation with each other as appropriate to individual plans and projects. Effective mitigation of adverse effects on Natura 2000 sites and species can only take place at Stage 3, when those effects have been fully recognised, assessed and reported in the Stage 2 assessment – it is then a matter for the Competent Authority, on the basis of consultation, to determine what type and level of mitigation is appropriate, and how to ensure that it is monitored and implemented over the life-time of the plan or project.

DG Environment Guidance indicates that the Stage 3 determination of whether alternative solutions exist or not is carried out by the Competent National Authority, when the Stage 2 assessment has indicated that adverse effects are likely. This assessment also takes the cumulative impacts of other projects and plans into account. Stage 3 is required to provide an objective assessment of whether alternative solutions to the plan or project exist, which would avoid any adverse impacts on the integrity of a Natura 2000 site or sites. In completing a Stage 3 assessment, DG Environment guidance indicates that “it rests with the national authorities to make the necessary comparisons between alternative solutions. It should be stressed that the reference parameters for such comparisons deal with aspects concerning the conservation and maintenance of the integrity of the site and of its ecological functions. In this phase, therefore, other assessment criteria, such as economic criteria, cannot be seen as overruling ecological criteria”. Alternative solutions are considered in the context of the project or plan objectives, and should include ‘do nothing’ and demand management solutions. The Stage 3 assessment should be documented, including the process of the final decision as to whether alternative solutions are available. If alternative solutions have been identified that will either avoid any adverse impacts, or result in less severe impacts on the Natura 2000 site, it will be necessary to assess their potential impact by recommencing the Article 6 assessment at Stage 1 or Stage 2. If it can be reasonably and objectively concluded that no alternative solutions exist, it will be necessary to proceed to Stage 4 of the Article 6 assessment.

### Stage 4 Assessment.

A Stage 4 assessment is carried out by the National Authorities in consultation with the European Commission. Guidance on the issues arising was published by the Commission in January 2007, this should be referred to in the event of a requirement for an Article 6 Stage 4 assessment (Guidance document on Article 6 (4) of the ‘Habitats Directive’ 92/43/EEC). Additional relevant considerations can be found in the Findings of the European Court of Justice in Case C-418/04 brought against Ireland by the European Commission (http://curia.europa.eu ).

All stages of the Article 6 assessment require consultation with the National Parks and Wildlife Service, by plan and project proponents, and by the other Competent Authorities and agencies relevant to the particular project/plan and its location.

## 7. MANAGEMENT AND MEASURES.

This Section identifies management, monitoring and other measures necessary to mitigate against the significant pressures impacting on, or likely to impact on habitats and species protected under the Habitats Directive. The measures are identified in each section and are further tabulated and provided in Appendix 12 and in the attached spreadsheet.

## 7.1. Identification of reasons for unfavourable conservation status

Unfavourable conservation status is currently noted for one or more water dependent habitats or species in a total of 86 SACs nationally. The information included in the SAC_Water_Dependency database is taken from the Article 17 reporting of conservation status of Habitats Directive Annex I and II listed habitats and species, which has been completed recently by NPWS (see Section 2.2.2), and also from expert opinion within NPWS.

The SAC_Water_Dependency GIS database contains two fields relating to unfavourable conservation status: UCS_Issue, and Water_Body_at_UCS.

Currently, there is relatively little site specific information on the conservation status of many of the water dependent habitats and species, but the available data refer to species which are particularly sensitive to water status, notably the two species of freshwater pearl mussel Margaritifera, and three species of whorl snail Vertigo. The site-specific information available relates principally to the following water dependent habitats and species:

Habitat:

Coastal lagoons

Species:

Najas flexilis (an aquatic plant occurring in lakes)

Vertigo angustior (a ground water dependent snail)

Vertigo geyeri (a ground water dependent snail)

Vertigo moulinsiana (a ground and surface water dependent snail)

Margaritifera margaritifera (freshwater pearl mussel occurring in rivers)

Margaritifera durrovensis (freshwater pearl mussel occurring only in the River Nore)

Some information on conservation status is available for other SACs from best expert opinion; these relate to lakes, turloughs and dune slacks. Unfavourable conservation status issues are currently noted for one or more water dependent habitats or species in a total of 86 SACs nationally. Water Framework Directive measures are required under the Habitats Directive to address the water status issues arising in these protected areas. The relevant fields from the SAC Water Dependency database are reproduced in Appendix 8.

The UCS_Issue field lists the water dependent habitat and/or species features assessed as being at unfavourable conservation status within each SAC. In some cases, the reasons for the unfavourable status have not been identified yet; and it is not clear whether a water status issue related to human activity arises, or whether the issue relates to natural change, in which case remedial measures may still be required. In these cases, the species or habitat at unfavourable status is noted by name but no further comment is made (e.g. Moneen Mountain SAC, site code 000054; UCS_Issue: Coastal Lagoon Muckinish lake). It is a matter for NPWS to continue to survey, review and address the issues contributing to unfavourable status; in some instances there may not be sufficient information on the individual site for the habitat or species, and further survey is required to clarify the situation. In other cases unfavourable status may be due to land use and management issues which are not related to water status. Note, however, for both fresh water pearl mussel species M. margaritifera and M. durrovensis, water quality and related habitat deterioration is confirmed as the issue causing unfavourable conservation status. Specific measures are discussed below.

7.1.1. Measures by NPWS to clarify whether water status is an issue:

- Continued monitoring and survey by NPWS to confirm conservation status

- Clarification of location of the features at unfavourable conservation status

- Implementation of land management measures within protected areas

- Notification of appropriate agencies if water status issues are found to arise

Where unfavourable conservation status arises as a result of local land management issues within SACs, Local Authorities and other agencies involved in implementing the Water Framework Directive should be aware that water status issues may arise cumulatively from existing pressures in these protected areas, and that further investigation may be required.

In some SACs where ground water dependent habitats and species are present, there may be a complex interaction between land management issues and existing or proposed abstractions. For example, if a fen habitat is under-grazed or not grazed at all, the area may tend to be colonised by trees and scrub which will have the effect of reducing the level of the water table locally, to the extent that the typical fen species of plants and animals will reduce in abundance and may eventually disappear. Land management issues and abstraction issues will have a cumulative effect on conservation status in these cases, and the application of measures to return the habitat or species to favourable status will require all relevant issues to be addressed in an integrated way with the involvement of all interested parties.

Where unfavourable conservation status has been linked to water status issues, the issue is noted in the UCS_Issue field; e.g. Clonakilty Bay SAC, Site Code 000091; UCS_Issue: Coastal Lagoons Clogheen/White's Marsh and Inchydoney highly eutrophic and with hydrographic impacts. In most instances the issues arising are either eutrophication, water abstraction or other hydrological pressures, or a combination of these. The source of eutrophication impacts is likely to vary between SAC sites and the specific location of the habitat or species at unfavourable status within the site, and will include the cumulative impacts of agriculture and/or forestry, diffuse inputs from small-scale WWTP and dispersed housing, and point discharges from WWTP and possibly other point discharges also. Some examples are given below.

Note the small area of

surface water present

on the day the aerial

photograph was taken

EPA mapped lake water body

SAC 000051

Includes Ordnance Survey of Ireland data

reproduced under OSI license No.

2003/07CCMA/Galway Co. Council

Newmarket on Fergus, Co. Clare. The shaded areas are Lough Gash (lake SEG_CD 27_116), and Lough Gash Turlough SAC (Site Code 000051), the SAC designation being somewhat larger than the mapped lake (a turlough habitat is defined by the extent of wetland vegetation associated with the basin, and the SAC may also include adjoining habitats and/or a buffer zone). Newmarket on Fergus WWTP discharges into a small stream flowing into the eastern side of the turlough basin; this is likely to be the main source of nutrient loading to the turlough, which is assessed as being at unfavourable conservation status with eutrophication noted. A WWTP discharge to a turlough basin is also likely to constitute a risk to groundwater status. The 2000 orthophotography shows a road under construction adjoining the southern end of the SAC.

The Water_body_at_UCS field in the SAC_Water_Dependency GIS database provides cross-referencing to other GIS map layers developed for the purposes of implementing the Water Framework Directive, and uses the SEG_CD or other relevant code. It refers to lakes, river sections, transitional and coastal water bodies, ground water bodies where these have been mapped, and to individual databases where there are too many codes to be listed individually, e.g all Margaritifera populations, coastal lagoons. It should be noted that many surface water features mapped as lakes are in fact turloughs with direct connection to ground water bodies through sinks and swallow holes (refer to draft turloughs consolidated database, now available from NPWS on request). Some fens are also mapped as lakes. Coastal lagoons are in some instances mapped as lakes and also as transitional water bodies.

Bridge Lough coastal lagoon, priority Annex 1 listed water dependent habitat within Galway Bay Complex SAC Site Code 000268. This is a karst lagoon (thus with ground water inputs), which is assessed as being moderately eutrophic, and at unfavourable conservation status. This lagoon is mapped as transitional water body WE_180_0200. Ground water vulnerability mapping of the lagoon area is mainly E and X. Diffuse pressures are likely to be the main factors leading to unfavourable conservation status at this site.

Ground water vulnerability categories E and X both refer to extreme vulnerability. E indicates a soil depth over bedrock of less that 1 metre. X indicates a soil depth over bedrock of 3 metres or less.

The lagoon is indicated as a point location (amber), the full extent of the habitat is the brown area extending around and to the west of the point location.

Includes Ordnance Survey of Ireland data reproduced under OSI license No. 2003/07CCMA/Galway Co. Council

Where abstraction is noted as an issue implicated in unfavourable conservation status, reference has been made to the National Abstractions Register, and the code of the abstraction(s) noted in the Water_body_at_UCS field. It should be noted that not all existing abstractions are listed in the register, and that there may be multiple abstractions affecting an individual SAC, even if they are not all located within it.

In other instances there may be a number of ground water bodies in the vicinity of the feature at unfavourable conservation status, and it is not immediately apparent which ones are relevant. In dune slack habitats, the ground water body supporting the habitat may be quite small and may not have been mapped as an individual feature (e.g. North Bull Island in Dublin Bay). This issue is also relevant to turloughs, where there may be karst flow at different depths, or a perched water table in part of the turlough basin.

Registered abstractions

Part of Buckroney-Brittas dunes and fen SAC

Buckroney fen

Stream subject to unregistered

Abstraction

Dune slack habitats 2160 and 2170

Includes Ordnance Survey of Ireland data reproduced under OSI license No. 2003/07CCMA/Galway Co. Council

Buckroney-Brittas dunes and fen SAC in Co. Wiklow (Site Code 000729).

In this example ground water dependent dune slack habitats are assessed as being at unfavourable conservation status, with abstraction noted as a likely cause, and with alkaline fen habitat possibly adversely affected also. An unregistered abstraction associated with the golf course is a possible contributing factor, and is recommended for inclusion in further investigative monitoring as a basis for measures under the Habitats Directive at this site.

7.1.2.   Measures by River Basin Management Planners and implementing agencies to clarify how water status contributes to unfavourable conservation status:

- Update National Abstractions Register

- Identify alternative solutions where existing abstractions are implicated in causing unfavourable conservation status

- Use existing analyses of risks and pressures in relevant catchments to identify the suite of WFD Measures that will contribute to resolving unfavourable conservation status for individual SACs. Include examination of ortho-photography in the review. LA Biodiversity Officers should be involved.

## 7.2. Remedial Measures

- Consult with NPWS with results of Measure 7.1.2 above, identify additional risks and sensitivities, and any additional Measures relevant to individual sites/features at unfavourable conservation status, including investigative/operational monitoring needs

- Document plans specific to each SAC unfavourable conservation status issue, with reference to Sections 5 and 6 of this guidance, submit to and  implement in consultation with NPWS

## 7.3. Specific measures

### 7.3.1. Measures for Margaritifera species:

A total of 28 SACs are at unfavourable conservations status for freshwater pearl mussels; these relate to 19 SACs where M. margaritifera is present as a Qualifying Interest (some of which have also have populations which were not originally noted as a qualifying interest). One of these 19 SACs also holds the sole remaining population of  M. durrovensis. Other populations of M. margaritifera also occur in at least partly in 9 other SACs; the rapid assessment survey currently in progress for NPWS will yield additional information about these.

Objectives for Margaritifera are expected to be formalised in European Communities Environmental Objectives (Freshwater Pearl Mussel) Regulations by the Minister for the Environment, Heritage and Local Government in early 2009. The sub-basin plans referred to in Section 5 above will be set up under these Regulations. These will apply to a total of 27 individual ‘qualifying’ populations of M. margaritifera, and the single remaining population of M. durrovensis.

Populations of M. margaritifera which will not be covered by the sub-basin plans referred to above are still subject to legal protection under both the Habitats Directive and national legislation (European Communities (Natural Habitats) Regulations). The following measures are recommended for these populations:

- GIS mapping of the currently known extent of currently non-qualifying M. margaritifera populations onto the EPA river sections database, including any distribution updates available from the  rapid assessment survey during 2008, with annual updates in 2009 and 2010 with any additional distribution data – this database simply records known presence of the species.

- Notification and distribution by NPWS of the GIS database to all relevant Local Authorities and other agencies including the EPA, DAFF, Forest Service, Coillte, COFORD, Central Fisheries Board and Regional Fisheries Boards, so that Margaritifera populations can be taken into account as sensitive receptors in the development of River Basin Management Plans, and in the consideration of future plans and projects

- Notification by NPWS of all Margaritifera monitoring transect locations to other monitoring agencies, to avoid any accidental damage by them

- Review by River Basin management planners of water status, risks and pressures, in catchments associated with Margaritifera populations, in order to assist NPWS in identifying the more readily recoverable populations, and in identifying potential translocation rivers for M. durrovensis

- On-going identification by NPWS of populations for which there is evidence that existing pressures are such that populations may be recoverable, with a view to applying appropriate conservation measures to them

- Investigative monitoring of river water quality parameters in association with NPWS rapid assessment monitoring

- Review controls on use and disposal of pesticides including sheep-dip

- Notification and circulation of the Margaritifera species action plan when this has been completed by NPWS

### 7.3.2. Measures for Vertigo species

Vertigo species are noted for very large annual fluctuations in individual populations, so that assessment of conservation status is most reliable when population data are available over a number of years and trends can be reliably established.

- Assessment of known Vertigo populations by NPWS is on-going; it is recommended that assessments are updated annually to 2010 to inform Programmes of Measures for these species.

- Map the locations of Vertigo populations in GIS, identify their relevant groundwater body(s) and delineate the zone of contribution, notify and circulate database to the relevant implementing agencies

- Assess requirements for remedial measures and implement

- Review controls on use and disposal of pesticides including sheep-dip

## 7.4. Avoidance of adverse impacts

These measures apply to all SACs and SPAs, irrespective of conservation status assessment. Note that relevant national legislation is under review currently.

7.4.1.   Prevention of adverse impacts through planning, licensing and grant- aiding procedures

- Refer to Section 6 for guidance on Article 6 Appropriate Assessment of plans and projects in or potentially affecting SACs and SPAs

- Refer to Sections 3 and 5 for information on the sensitivities of water dependent habitats and species to water quality and hydrological pressures

- Consult with NPWS

7.4.2.   Prevention of adverse impacts through development and implementation of Codes of Practice

  - It is recommended that a Code of Good Practice is developed for construction activity in and adjoining watercourses. Bad practice is widespread nationally and results in significant unnecessary and avoidable adverse impacts on watercourses, including siltation impacts

  - It is recommended that a code of good farming practice is developed for the management of riparian zones, and the management of ditches and field drainage systems, with the aim of reducing siltation impacts on watercourses

## 7.5. Administrative - Database measures

7.5.1. Register of Protected Areas update for SACs

- The Register of Protected Areas requires updating in respect of SACs designated under the Habitats Directive for Annex 1 listed water dependent habitats and Annex 2 species. The relevant field in the MapInfo SAC_Water_Dependency database is WD_H_S. This is a logical field (true/false), to be queried with the expression WD_H_S = “T”

- This database must be included in the EPA ENVision Online Mapviewer WFD Protected Areas

- It is recommended that the SAC_Water_Dependency database reverts to NPWS for updating, with any updates notified annually to the EPA, which should then notify and disseminate updates to all agencies involved in the implementation of the Water Framework Directive

7.5.2 Register of Protected Areas update for SPAs

- The Register of Protected Areas shown on the EPA ENVision Online Mapviewer WFD Protected Areas is out of date in respect of SPAs; additional sites have been designated since 2005, and boundaries revised for existing sites, and these must be included

- NPWS should clarify which SPAs are designated for water dependent bird species, although it is noted that most sites are listed for internationally important wintering waterfowl populations and/or seabird breeding colonies. It is also noted that a major revision of the SPA network is in progress

- NPWS should develop a second tier (shadow list) database of water dependent habitats and species occurrence in conservation sites designated under national legislation (NHAs, designated principally for bogs). It is also recommended that NPWS progresses with the formal designation of pNHAs to NHA status, accompanied by a boundary review to ensure that water dependent habitats and species are included, together with appropriate buffer zones

7.5.3. Database cross-referencing measures

Key databases relevant to the implementation of the Water Framework Directive do not cross-refer effectively at present:

- NPWS databases for water dependent habitat and species (e.g. lakes, fens, Margaritifera species, etc.) should include fields cross-referencing individual habitat features and species distribution to EPA water body codes for the river sections, lakes, transitional waters, coastal waters, and ground water bodies with which the habitat and species are associated

- EPA databases on water body status should include fields cross-referencing individual mapped features to SAC Site Code, SPA Site Code and NPWS Site Name, where the features occurs within the boundaries of an SAC or SPA. Fields should also be included to cross-reference lakes to Habitats Directive Annex 1 listed lake habitats and turloughs, and updated as more habitat information becomes available from NPWS

- The National Abstractions Register should include fields cross-referencing abstractions to the Site Code and Site Name of SACs and SPAs (and the second tier NHAs and pNHAs) they are located in

7.5.4.   Establishment of a Register of databases relevant to the implementation of the Water Framework Directive.

- It is recommended that a central Register of relevant databases is established, to include reference to all Water Framework Directive – relevant databases developed and maintained by all relevant agencies including NPWS, EPA, GSI, Central and Regional Fisheries Boards, Marine Institute, Forest Service, Local Authorities. This could be done under INSPIRE

- All agencies holding relevant databases should establish procedures for regular update and notification of updates, and establish procedures for providing these data in GIS together with the associated metadata. Guidance on how data are to be used may also be necessary

- Agencies displaying information (via on-line map viewers) that is provided and up-dated by other agencies, should establish procedures for updating this information, and an on-line means of identifying which version is currently displayed

## 7.6. Measures to encourage appropriate land uses within protected areas for water dependent habitats and species

- A review of Notifiable Activities by NPWS is in progress. It is recommended that these include review of the findings of all NPWS and other relevant monitoring programmes, e.g. Commonage Framework Plan monitoring.

- Administrative measures relating to the implementation of approved farm plans are also recommended, to ensure that NPWS objectives are fully reflected in the plans.

- Supports for organic farming within Protected Areas are recommended.

- Nutrient and silt loading issues should be addressed with regard to the ecological requirements of individual Annex listed habitats and species present in individual Protected Areas, with provision for site-specific variation in soils and underlying geology, slope, livestock type, and seasonal restrictions on grazing and/or mowing regimes

- Ex-situ issues and associated measures identified by NPWS as required to assist in returning individual Natura 2000 sites to favourable conservation status should be notified to the EPA and the relevant Local Authority

## 7.7. Measures for High Status Sites

In addition to the requirements to protect and restore for  Habitats Directive Annex 1 and 2 listed water dependent habitats and species (WFD Annex IV Protected Areas), there are a number of other waters for which High Status must be protected under the Water Framework Directive. These include the following:

Table 7.1.  High Status water bodies and high status sites objectives

| Water body or high status site | Objective |
| --- | --- |
| River water bodies assigned interim high status classification by the EPA | Protect |
| Lake water bodies assigned interim high status classification by the EPA | Protect |
| Transitional water bodies assigned interim high status classification by the EPA | Protect |
| Coastal water bodies assigned interim high status classification by the EPA | Protect |
| Char lakes with existing populations | Protect |
| Catchment areas of WFD or EPA Biological Q assessment sites at High Status (Q5 or Q 4.5) | Protect |

The objective of the following measures is

- to link the requirements for High Status Sites listed under the Water Framework Directive, with the requirements of the Habitats Directive with regard to favourable conservation status, where relevant

- to identify water bodies or catchment areas of monitoring locations which are close to undisturbed reference conditions, and to protect them from deterioration.

The rationale is therefore similar to the requirements of the Habitats Directive, although the detail by which High Status Sites and SACs are identified, and protective measures are applied and monitored under the two Directives are substantially different.

Many of the measures under the Habitats Directive which are listed and discussed above are relevant to High Status Sites also. Appendix 12 of this guidance lists these measures, and ascribes codes to them which will be used in GIS to indicate individual water bodies and locations to which suites of measures will apply. A column in  Appendix 12 ascribes codes to those measures which are relevant to High Status Sites. Three additional measures for High Status Sites are given below. These are also included in Appendix 12.

For high status water bodies or high status sites (Q5, Q4.5) which are not designated under the Habitats Directive the water quality objectives set out in the Draft European Communities Environmental Objectives (Surface Waters) Regulations 2008, “Schedule 5, Criteria for calculating surface water ecological status and ecological potential and Schedule 6,  The environmental quality standards for priority substances and certain other pollutants to apply for the purpose of assigning chemical status must be strictly adhered to. The criteria are set out in Appendix 13. It should be noted that where areas are designated under the Habitats Directive then water quality EQS standards as set out in the draft Regulations may not be stringent enough to meet the requirements (e.g. Dystrophic lakes (3160) and Oligotrophic lakes (3110), and possibly the oligotrophic end of the range of Hard Oligo-mesotrophic waters with benthic vegetation of Chara spp (3140)). Additional measures for High Status Sites are given below. These are also included in Appendix 12.

- Review sites for habitats and species which are at favourable conservation status under the Habitats Directive, and which are not covered by a conservation designation currently; apply an appropriate conservation designation under EU and/or national legislation

- Ensure that all water bodies under consideration for listing as High Status Sites under the Water Framework Directive are at favourable conservation status for any Habitats Directive Annex 1 water dependent habitats and/or Annex 2 species known to be associated with them

- Identify water bodies at high status, water bodies with WFD objective of restoring to  high status catchment areas of monitoring sites at high status ((Q5 and Q4.5) in Regional, County and Local Development Plans

- Ensure that the appropriate high environmental quality standards are applied to all water bodies that would qualify for High Status under the Water Framework Directive, but where status has been down-graded to Moderate because an Annex 1 water dependent habitat or Annex 2 water dependent species is not at favourable conservation status under the Habitats Directive

- Ensure that the appropriate high environmental quality standards, as set out in the Draft Environmental Quality Objectives Regulations are applied to all high status water bodies and catchment areas of high status monitoring sites (Biological Quality rating Q4 and Q4.5)

- Prohibit point and diffuse source discharges liable to cause water pollution except where such discharges are subject to prior authorisation or general binding rules. Discharge authorisations must lay down emission limits that aim to achieve the environmental objectives/quality standards specified in the draft regulations.

## BIBLIOGRAPHY.

Bamber, R.N, Gilliland, P.M. and Shardlow, M.E.A. (2001). Saline lagoons: a guide to their management and creation (interim version). ISBN 1 85716573 X Peterborough, English Nature.

Bobbink, Roland; Mike Ashmore, Sabine Braun, Walter Flückiger and Isabel J.J. Van den Wyngaert (2002). Empirical nitrogen critical loads for natural and semi-natural ecosystems: 2002 update. Report to UN Ecomonic and Social Council, Economic Commission for Europe; Executive body for the Convention on long-range transboundary air pollution.

Curtis, Tom, Sheila Downes and Bernadette Ní Chatháin (2006). North-South Share Project: Register of Protected Areas. Report on the ecological requirements of water dependent habitats and species designated under the Habitats Directive. RPS Consulting Engineers

DG Environment Guidance on the provisions of the Habitats Directive and the management of Natura 2000 sites, available at http://ec.europa.eu/environment/nature/natura2000/management/guidance en.htm :

- Methodological guidance on the provisions of Article 6(3) and (4) of the Habitats Directive 92/43/EU. DG Environment © European Communities 2002.

- Guidance document on Article 6 (4) of the ‘Habitats Directive’ 92/43/EEC.(January 2007)

- Managing Natura 2000 sites: The provisions of Article 6 of the ‘Habitats’ Directive 92/43/EEC. ISBN 92-828-9048-1 © European Communities, 2000.

Environmental Consultancy Services (1997). Williamstown Environmental Study. Report to Engineering Services, Office of Public Works.

Fossitt, Julie A. (2000) A guide to habitats in Ireland. The Heritage Council

Kilroy, Garrett, and Fiona Dunne (2008) A Framework for the Assessment of Groundwater-Dependent Terrestrial Ecosystems under the Water Framework Directive. Environmental Research Centre. ISBN: 978-1-84095-005-2 © EPA

National Parks and Wildlife Service, 2008. The Status of EU Protected Habitats and Species in Ireland. Available on www.npws.ie

NRA Guidelines on the Treatment of Air Quality During the Planning and Constriction of National Road Schemes.

Ó Riain, Gearóid Katherine Duff and Maria Long (2005). Water Framework Directive – Water Status: identifying and ranking of Nature Conservation Designated Areas (2005). Prepared by Compass Informatics Limited and Natura (Environmental Consultants) Ltd. Environmental RDTI Programme 2000-2006 (2002-W-DS-10).

Oliver (2005). Seasonal changes and biological classification of Irish coastal lagoons. G. A. Oliver,Ph.D. Thesis 2005, Department of Zoology, University College Dublin.

Oliver (2007). Conservation Status Assessment Report: Coastal Lagoons (1150). March 2007. Report to NPWS.

WFD CIS Guidance Document No 12 – Horizontal Guidance on the Role of Wetlands in the Water Framework Directive. © European Communities 2003. ISBN 92-894-6967-6

Note that the Article 17 conservation status assessment reports include extensive bibliographies

## APPENDIX 1: WATER DEPENDENT HABITATS

The following table is taken from O’Riain et al (2005), and incorporates subsequent amendments by NPWS. It lists the water dependent habitats recognised as occurring in Ireland. Habitats Directive Annex 1 listed habitats are given in the 1st and 2nd columns; priority listed habitats are bold-faced. The Fossitt/Heritage Council habitats broadly corresponding to each Annex 1 habitat are listed in columns 3 and 4. Water dependent Heritage Council habitat types which do not correspond to Annex 1 listed habitats are also included for reference: these occur widely in SPAs and pNHAs.

Main water sources are c: coastal; t: transitional; s:  surface; g: ground;

and p: precipitation.

| EU Code | EU Annex I habitats (Natura 2000 code) (* = priority type) | Fossitt Code | Fossitt habitat type | Main Water Source ** |
| --- | --- | --- | --- | --- |
| MARINE AND COASTAL HABITATS |  |  |  |  |
|  |  |  | Sublittoral sediments |  |
| 1110 | Sandbanks which are slightly covered by sea water all the time | SS1 | Infralittoral gravels and sands | c, t |
|  |  | SS2 | Infralittoral muddy sands | c, t |
|  |  | SS3 | Infralittoral muds | c, t |
|  |  | SS4 | Infralittoral mixed sediments | c, t |
|  |  | SS5 | Circalittoral gravels and sands | c, (t) |
|  |  | SS6 | Circalittoral muddy sands | c, (t) |
|  |  | SS7 | Circalittoral muds | c, (t) |
|  |  | SS8 | Circalittoral mixed sediments | c, (t) |
| 1130 | Estuaries | MW4 | Estuaries | t,c,s |
|  |  |  | Littoral sediments |  |
| 1140 | Mudflats and sandflats not covered by sea water at low tide | LS2 | Sand shores | c, t |
|  |  | LS3 | Muddy sand shores | c, t |
|  |  | LS4 | Mud shores | c, t |
|  |  | LS5 | Mixed sediment shores | c, t |
| 1150 | *Coastal lagoons | CW1 | Lagoons and saline lakes | t, c, s, g |
| 1160 | Large shallow inlets and bays | MW2 | Sea inlets and bays | c |
|  |  |  | Marine water body |  |
|  |  | MW1 | Open marine water | c |
|  |  | MW3 | Straits and sounds | c, (t) |
| 1170 | Reefs |  | Littoral rock |  |
|  |  | LR1 | Exposed rocky shores | c, (t) |
|  |  | LR2 | Moderately exposed rocky shores | c, (t) |
|  |  | LR3 | Sheltered rocky shores | c, (t) |
|  |  | LR4 | Mixed substrata shores | c, (t) |
|  |  |  | Sublittoral rock |  |
|  |  | SR1 | Exposed infralittoral rock | c |
|  |  | SR2 | Moderately exposed infralittoral rock | c |
|  |  | SR3 | Sheltered infralittoral rock | c, (t) |
|  |  | SR4 | Exposed circalittoral rock | c |
|  |  | SR5 | Moderately exposed circalittoral rock | c |
|  |  | SR6 | Sheltered circalittoral rock | c, (t) |
|  |  | Note there is no Fossitt habitat category for cold water coral reefs, though the sublittoral rock habitats may contain examples |  |  |
| 8330 | Submerged or partially submerged sea caves | LR5 | Sea caves | c |
| COASTAL HABITATS WITH TERRESTRIAL VEGETATION |  |  |  |  |
| 1210 | Annual vegetation of drift lines | LS1 | Shingle and gravel shores | c, (t) |
|  |  | LS2 | Sand shores | c, (t) |
| 1220 | Perennial vegetation of stony banks | CB1 | Shingle and gravel banks | c, (t) |
| 1230 | Vegetated sea cliffs of the Atlantic and Baltic coasts | CS1 | Rocky sea cliffs | c, (t) |
|  |  | CS2 | Sea stacks and islets | c, (t) |
|  |  | CS3 | Sedimentary sea cliffs | c, (t) |
| 1310 | Salicornia and other annuals colonising mud and sand | CM1 | Lower salt marsh | c,t |
| 1330 | Atlantic salt meadows (Glauco-Puccinellietalia maritimae) | CM1, CM2 | Lower salt marsh Upper salt marsh | c,t c,t,s,g |
| 1410 | Mediterranean salt meadows (Juncetalia maritimi) | CM2 | Upper salt marsh | c,t,s,g |
| 1420 | Mediterranean and thermo-Atlantic halophilous scrubs (Sarcocornetea fruticosi) | CM1 | Lower salt marsh | c,t |
| 2110 | Embryonic shifting dunes | CD1 | Embryonic dunes | c, (t) |
| 2120 | Shifting dunes along the shoreline with Ammophila arenaria (“white dunes”) | CD2 | Marram dunes | c, (t) |
| 2130 | *Fixed coastal dunes with herbaceous vegetation (“grey dunes”) | CD3 | Fixed dunes | c,(t) |
| 2140 | *Decalcified fixed dunes with Empetrum nigrum |  |  |  |
| 2150 | *Atlantic decalcified fixed dunes (Calluno-Ulicetea) |  |  |  |
| 2170 | Dunes with Salix repens ssp. argentea (Salicion arenariae) | CD5 | Dune slacks | g, c, (t) |
| 2190 | Humid dune slacks |  |  |  |
| 21A0 | Machairs (*in Ireland) | CD6 | Machair | c, g, s |
| FRESHWATER LAKES, TURLOUGHS |  |  |  |  |
| 3110 | Oligotrophic waters containing very few minerals of sandy plains (Littorelletalia uniflorae) | FL2 | Acid oligotrophic lakes | s, g |
| 3130 | Oligotrophic to mesotrophic standing waters with vegetation of the Littorelletea uniflorae and/or of the Isoëto-Nanojuncetea | FL2 FL4 | Acid oligotrophic lakes Mesotrophic lakes | s, g |
| 3140 | Hard oligo-mesotrophic waters with benthic vegetation of Chara spp. | FL3 | Limestone/marl lakes | g, s |
|  |  | FL4 | Mesotrophic lakes | s, g |
| 3150 | Natural eutrophic lakes with Magnopotamion or Hydrocharition-type vegetation | FL4 FL5 | Mesotrophic lakes Eutrophic lakes | s, g |
| 3160 | Natural dystrophic lakes and ponds | FL1 | Dystrophic lakes | s, (g) |
| 3180 | *Turloughs | FL6 | Turloughs | g, s |
|  |  | FL7 | Reservoirs | s |
|  |  | FL8 | Other artificial lakes and ponds | s |
| FRESHWATER WATERCOURSES |  |  |  |  |
| 3260 | Watercourses of plain to montane levels with the Ranunculion fluitantis and Callitricho-Batrachion vegetation | FW1 | Eroding/upland rivers | s, g |
|  |  | FW2 | Depositing/lowland rivers | s, g |
| 3270 | Rivers with muddy banks with Chenopodion rubri p.p. and Bidention p.p. vegetation | FL6 | Turloughs | s, g |
|  |  | FW2 | Depositing/lowland rivers | s, g |
|  |  | FW3 | Canals | g, s |
|  |  | FW4 | Drainage ditches | s, g |
| SPRINGS AND FENS |  |  |  |  |
| 7220 | *Petrifying springs with tufa formation (Cratoneurion) | FP1 | Calcareous springs | g |
| 7210 | *Calcareous fens with Cladium mariscus and species of the Caricion davallianae | PF1 | Rich fen and flush | g, s |
| 7230 | Alkaline fens | PF1 | Rich fen and flush | g, s |
|  |  | FP2 | Non-calcareous springs | g |
|  |  | PF2 | Poor fen and flush | g, s |
| HEATH AND BOG |  |  |  |  |
| 4010 | Northern Atlantic wet heaths with Erica tetralix | HH3 FP2 PF2 | Wet heath Non-calcareous springs Poor fen and flush | s, p, (g) g g |
| 7110 | *Active raised bogs | PB1 PF2 | Raised bog Poor fen and flush | p, s, (g) g |
| 7120 | Degraded raised bogs still capable of natural regeneration | PB1 PF2 PB5 | Raised bog Poor fen and flush Cutover bog | p, s, (g) g p, s, (g) |
| 7130 | Blanket bog (*if active bog) | PB2 PB3 FP2 PF2 PB4 PB5 | Upland blanket bog Lowland blanket bog Non-calcareous springs Poor fen and flush Cutover bog Eroding blanket bog | p, s, (g) p, s, (g) g g p, s, (g) p, s, (g) |
| 7140 | Transition mires and quaking bogs | PF3 | Transition mire and quaking bog | s, p, (g) |
| 7150 | Depressions on peat substrates of the Rhynchosporion |  | Can occur in PB1, PB2, PB3 and PB4 |  |
| 91D0 | *Bog woodland | WN7 | Bog woodland | s, p, g |
| SWAMPS |  |  |  |  |
|  |  | FS1 | Reed and large sedge swamps | s, g |
| 6430 | Hydrophilous tall herb fringe communities of plains and of the montane to alpine levels | FS2 | Tall-herb swamps | s, g |
| GRASSLAND AND MARSH |  |  |  |  |
| 6410 | Molinia meadows on calcareous, peaty or clayey-silt-laden soils (Molinion caeruleae) | GS4 | Wet grassland | s, g |
|  |  | GS4 | Wet grassland |  |
| WOODLAND AND SCRUB |  |  |  |  |
| 91E0 | *Alluvial forests with Alnus glutinosa and Fraxinus excelsior (Alno-padion, Alnion incanae, Salicion albae) | WN4 WN5 WN6 | Wet pedunculate oak-ash woodland Riparian woodland Wet willow-alder-ash woodland | s, g s, g s, g |
| EXPOSED ROCK/DISTURBED GROUND |  |  |  |  |
| 8310 | Caves not open to the public | EU1 | Non-marine caves | g |

## APPENDIX 2 –   HABITATS DIRECTIVE ANNEX LISTED  WATER-DEPENDENT FAUNA SPECIES IN IRELAND

Fauna species (excluding birds, which are listed in Annex 1 of the Birds Directive) occurring in Ireland which are protected under:

- Annex II, Annex IV and Annex V of the Habitats Directive (EU HD); note that the SAC_Water_Dependency table lists Annex 2 species only

- the Wildlife Act (WA).

  - *Note that Lesser horseshoe bats are included for information only, and only where cave sites are used as roosts

  - **Arctic char have been noted because of their threatened status, high lake water quality requirement and hence usefulness as an indicator species, and vulnerability to water abstraction; this species is not currently listed in Annexes to the Habitats Directive

  - *** In Ireland, Pollan occur in five lakes (Lough Neagh, Lower Lough Erne, Lough Derg, Lough Ree, and have recently been confirmed to occur in Lough Allen also)

  - Overall conservation status for Annex listed species is indicated for each species by colour: green= favourable, amber= unfavourable-inadequate, red= unfavourable-bad, grey= unknown

| Common name | Latin name | EU HD | WA |
| --- | --- | --- | --- |
| MAMMALS |  |  |  |
| Lesser horseshoe bat* | Rhinolophus hipposideros | II, IV | P |
| Grey Seal | Halichoerus grypus | II, V | P |
| Common Seal | Phoca vitulina | II, V | P |
| Bottle-nosed Dolphin | Tursiops truncatus | II, IV | P |
| Harbour Porpoise | Phocaena phocaena | II, IV | P |
| Otter | Lutra lutra | II, IV | P |
| FISH |  |  |  |
| River Lamprey | Lampetra fluviatilis | II, V | - |
| Brook Lamprey | Lampetra planeri | II | - |
| Sea Lamprey | Petromyzon marinus | II | - |
| Atlantic Salmon (freshwater only) | Salmo salar | II, V | - |
| **Arctic Char | Savelinus alpinus |  |  |
| Allis Shad | Alosa alosa | II, V | - |
| Twaite Shad | Alosa fallax fallax | II, V | - |
| Killarney Shad | Alosa fallax killarnensis | II, V | - |
| ***Pollan | Coregonus autumnalis | V |  |
| AMPHIBIANS |  |  |  |
| Natterjack Toad | Bufo calamita | IV | P |
| CRUSTACEANS |  |  |  |
| White-clawed Crayfish | Austropotamobius pallipes | II, V | P |
| INSECTS |  |  |  |
| Marsh Fritillary | Euphydryas aurinia | II | - |
| MOLLUSCS |  |  |  |
| Narrow-mouthed whorl snail | Vertigo angustior | II | - |
| Geyer’s whorl snail | Vertigo geyeri | II | - |
| Desmoulin’s whorl snail | Vertigo moulinsiana | II | - |
| Freshwater pearl mussel | Margaritifera margaritifera | II, V | P |
| Nore freshwater pearl mussel | Margaritifera durrovensis | II, V | P |

KEY EU HD = EU Habitats Directive. II = Annex II (of EU Habitats Directive) animal and plant species whose conservation requires the designation of SACs. IV = Annex IV animal and plant species of community interest in need of strict protection . V = Annex V animal and plant species whose taking in the wild and exploitation may be subject to management measures. WA = Wildlife Act 1976, and the Wildlife (Amendment) Act 2000. P = 	Animal species listed for protection in the Wildlife Act 1976 and Wildlife (Amendment) Act 2000

Species removed from the original O’Riain et al (2005)  list of water dependent fauna species in consultation with NPWS are the Kerry slug and Common lizard.

Note that whale and dolphin species not included in the list generally occur outside the RBD areas, but are of course water dependent and must be taken into account in the planning offshore construction/extraction activities

Measures for Natterjack toads are carried out under NPWS conservation planning, and involve the development of breeding ponds.

## APPENDIX 3 –   LIST OF ANNEX 2 WATER-DEPENDENT PLANT SPECIES IN IRELAND

Overall conservation status is indicated for each species by colour: green= favourable, amber= unfavourable-inadequate, red= unfavourable-bad, grey= unknown

| COMMON NAME | SCIENTIFIC NAME | EU HD |
| --- | --- | --- |
| Killarney Fern | Trichomanes speciosum | II, IV |
| Yellow Marsh Saxifrage | Saxifraga hirculus | II, IV |
| Slender Naiad | Najas flexilis | II, IV |
| Shining sicklemoss | Drepanocladus vernicosus | II |
| Petalwort | Petalophyllum ralfsii | II |

## APPENDIX 4 -  DEFINITION TABLE,  SAC WATER DEPENDENCY

Mapping of Special Areas of Conservation (SACs) in Ireland

SAC sites are mapped by NPWS, and are available from the NPWS website (www.npws.ie). Many individual SACs comprise a number of mapped polygons, either because they cross county boundaries, or the Qualifying Interests are specific features in the landscape which are separated by lands which are not of conservation importance, e.g Kilroosky Lough Cluster (Site Code 001786).  Currently there are some 1,850 SAC mapped polygons ascribed to 419 coastal and onshore SACs. For the purpose of this report, polygons referring individual SACs have been linked in the SAC_Water_Dependency database, because the data used in the attribute table refers to the entire area of each individual SAC.

Updates are notified regularly by NPWS, and are available in downloadable GIS files. To subscribe to the NPWS data mailing list, please contact NPWS_GIS@environ.ie or robert.ovington@environ.ie  Users of this database are advised to subscribe.

Fields: 104

| FIELDS SUMMARISING OVERALL WATER DEPENDENCY OF SACs |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| FIELD NAME | EXPLANATION |  |  |  |  |
| DESIG | Designation: SAC |  |  |  |  |
| COUNTIES | Counties within which SAC occurs |  |  |  |  |
| VER | Refers to the NPWS GIS updating process for the extent and boundary of sites subject to conservation designations. Updates to an individual site may only relate to a portion of that site within an individual county. The whole county file is updated to facilitate Local Authorities, and the VER entry indicates the version of the mapping used in this database. |  |  |  |  |
| RBD | River Basin District(s) in which the SAC occurs |  |  |  |  |
| SITECODE | Site Code allocated by NPWS. This field is indexed to the MapInfo map layer accompanying the browser table |  |  |  |  |
| SITE_NAME | Site Name allocated by NPWS |  |  |  |  |
| UCS_issue | Unfavourable Conservation status issue: indicates the habitat or species currently identified as being at unfavourable conservation status for water status reasons within an SAC, and names the water body requiring measures under the Habitats Directive where applicable |  |  |  |  |
| Water_body_at_UCS | EPA Code of water body at unfavourable conservation status (for fresh water pearl mussel species, refer to the Margaritifera databases). For abstractions, the National Abstractions Register code is given (note that relevant abstractions may not be included in the Register yet) |  |  |  |  |
| SAC | SAC formally adopted by European Commission (Y = yes) |  |  |  |  |
| cSAC | SAC fully notified but not yet formally adopted (Y = yes) |  |  |  |  |
| pNHA_1995 | SAC overlaps with a pNHA, boundaries may differ (Y = yes) |  |  |  |  |
| Nature_Reserve | SAC includes a Nature Reserve (Y = yes) |  |  |  |  |
| WD_H_S | Site includes a water dependent habitat or species as a qualifying interest (T = True, F = False) |  |  |  |  |
| WDH_Q | Site includes a water dependent habitat as a qualifying interest (T = True, F = False) |  |  |  |  |
| WDH_P | Site includes a water dependent habitat, other than as a qualifying interest (T = True, F = False) |  |  |  |  |
| WDS_Q | Site includes a water dependent species as a qualifying interest (T = True, F = False) |  |  |  |  |
| WDS_P | Site includes a water dependent species, other than as a qualifying interest (T = True, F = False) |  |  |  |  |
| Priority_WDH_Q | Site includes a priority listed water dependent habitat as a qualifying interest (T = True, F = False) |  |  |  |  |
| Priority_WDH_P | Site includes a priority listed water dependent habitat, other than as a qualifying interest (T = True, F = False) |  |  |  |  |
| Annex1_WDH_Q | Site includes an Annex 1 listed water dependent habitat as a qualifying interest (T = True, F = False) |  |  |  |  |
| Annex1_WDH_P | Site includes an Annex 1 listed water dependent habitat, other than as a qualifying interest (T = True, F = False) |  |  |  |  |
| Coastal_marine | Site includes coastal marine habitats as a qualifying interest (T = True, F = False) |  |  |  |  |
| Coastal_transitional | Site includes coastal transitional habitats as a qualifying interest (T = True, F = False) |  |  |  |  |
| Coastal_onshore | Site includes coastal onshore habitats as a qualifying interest (T = True, F = False) |  |  |  |  |
| Surface_water_dependent | Site includes surface water dependent habitats as a qualifying interest (T = True, F = False) |  |  |  |  |
| Groundwater_dependent | Site includes groundwater dependent habitats as a qualifying interest (T = True, F = False) |  |  |  |  |
| Precipitation_dependent | Site includes precipitation dependent habitats as a qualifying interest (T = True, F = False) |  |  |  |  |
| WATER DEPENDENT HABITAT DATA Q = PRESENT AS A QUALIFYING INTEREST; P = PRESENT |  |  |  |  |  |
| HABITAT NAME |  |  |  | HABITAT GROUP |  |
| _1110_Sandbanks_which_are_slightly covered by sea water all the time |  |  |  | Coastal marine |  |
| _1130_Estuaries |  |  |  | Coastal transitional |  |
| _1140_Mudflats_and_sandflats_not covered by sea water at low tide |  |  |  | Coastal transitional |  |
| _1150_Coastal_lagoons |  |  |  | Coastal transitional |  |
| _1160_Large_shallow_inlets_and bays |  |  |  | Coastal marine |  |
| _1170_Reefs |  |  |  | Coastal marine |  |
| _1210_Annual_vegetation_of_drift lines |  |  |  | Coastal onshore |  |
| _1220_Perennial_vegetation_of stony banks |  |  |  | Coastal onshore |  |
| _1230_Vegetated_sea_cliffs_of the Atlantic and Baltic coasts |  |  |  | Not included in a group |  |
| _1310_Salicornia_and_other_annuals colonising mud and sand |  |  |  | Coastal transitional |  |
| _1330_Atlantic_salt_meadows |  |  |  | Coastal transitional |  |
| _1410_Mediterranean_salt_meadows |  |  |  | Coastal transitional |  |
| _1420_Mediterranean_and_thermo-Atlantic halophilous scrubs |  |  |  | Coastal transitional |  |
| _2110_Embryonic_shifting_dune |  |  |  | Coastal onshore |  |
| _2120_Shifting_dunes_along_the shoreline with Ammophila arenaria |  |  |  | Coastal onshore |  |
| _2130_Fixed_coastal_dunes_with herbaceous vegetation (grey dunes) |  |  |  | Coastal onshore |  |
| _2140_Decalcified_fixed_dunes with Empetrum nigrum |  |  |  | Coastal onshore |  |
| _2150_Atlantic_decalcified_fixed dunes |  |  |  | Coastal onshore |  |
| _2170_Dunes_with_Salix_repens |  |  |  | Coastal onshore |  |
| _2190_Humid_dune_slacks |  |  |  | Coastal onshore |  |
| _21A0_Machairs_in_Ireland |  |  |  | Coastal onshore |  |
| _3110_Oligotrophic_waters containing very few minerals of sandy plains |  |  |  | Surface water dependent |  |
| _3130_Oligotrophic_to_mesotrophic standing waters with vegetation of the Littorelletea uniflorae and/or of the Isoёto-Nanojuncetea |  |  |  | Surface water dependent |  |
| _3140_Hard_oligo_mesotrophic waters with benthic vegetation of Chara spp. |  |  |  | Surface water dependent |  |
| _3150_Natural_euthrophic_lakes with Magnopotamion or Hydrocharition-type vegetation |  |  |  | Surface water dependent |  |
| _3160_Natural_dystrophic_lakes and ponds |  |  |  | Surface water dependent |  |
| _3180_Turloughs |  |  |  | Groundwater dependent |  |
| _3260_Water_courses_of_plain_to montane levels with the Ranunculion fluitantis and Callitricho-Batrachion vegetation |  |  |  | Surface water dependent |  |
| _3270_Rivers_with_muddy_banks with Chenopodium rubric p.p. and Bidention P.P vegetation |  |  |  | Surface water dependent |  |
| _4010_Northern_Atlantic_wet_heaths with Erica tetralix |  |  |  | Precipitation dependent |  |
| _6410_Molinia_meadows_on_calcareous, peaty of clayey-silt-laden soils |  |  |  | Not included in a group |  |
| _6430_Hydrophilous_tall_herb_fringe communities of plains and of the montane to alpine levels |  |  |  | Surface water dependent |  |
| _7110_Active_raised_bogs |  |  |  | Precipitation dependent |  |
| _7120_Degraded_raised_bogs still capable of natural regeneration |  |  |  | Precipitation dependent |  |
| _7130_Blanket_bog_active |  |  |  | Precipitation dependent |  |
| _7140_Transition_mires_and_quaking bogs |  |  |  | Precipitation dependent |  |
| _7150_Depressions_on_peat_substrates of the Rhynchosporion |  |  |  | Precipitation dependent |  |
| _7210_Calcareous_fens_with_Cladium mariscus and species of the Caricion davallianae |  |  |  | Groundwater dependent |  |
| _7220_Petrifying_springs_with tufa formation |  |  |  | Groundwater dependent |  |
| _7230_Alkaline_fens |  |  |  | Groundwater dependent |  |
| _8310_Caves_not_open_to_the_public |  |  |  | Groundwater dependent |  |
| _8330_Submerged_or_partly_submerged sea caves |  |  |  | Coastal marine |  |
| _91D0_Bog_woodland |  |  |  | Precipitation dependent |  |
| _91E0_Alluvial_forests_with_Alnus glutinosa and Fraxinus excelsior |  |  |  | Surface water dependent |  |
| WATER DEPENDENT SPECIES SUMMARY DATA |  |  |  |  |  |
| WDS_Q_2 |  | Site includes a water dependent species as a qualifying interest (T = True, F = False) |  |  |  |
| WDS_P_2 |  | Site includes a water dependent species, other than as a qualifying interest (T = True, F = False) |  |  |  |
| WD_Plant_Q |  | Site includes a water dependent plant species as a qualifying interest |  |  |  |
| WD_Fish_Q |  | Site includes a water dependent fish species as a qualifying interest |  |  |  |
| WD_Vertigo_Q |  | Site includes a water dependent Vertigo species as a qualifying interest |  |  |  |
| WD_Marine_mammal_Q |  | Site includes a water dependent marine mammal species as a qualifying interest |  |  |  |
| WATER DEPENDENT SPECIES DATA Q = PRESENT AS A QUALIFYING INTEREST; P = PRESENT |  |  |  |  |  |
| SPECIES CODE AND NAME |  |  | COMMON NAME |  | GROUP |
| _1102_Alosa_alosa |  |  | Allis shad |  | Fish |
| _1103_Alosa_fallax |  |  | Twaite shad |  | Fish |
| _1099_Lampetra_fluviatilis |  |  | River lamprey |  | Fish |
| _1096_Lampetra_planeri |  |  | Brook lamprey |  | Fish |
| _1095_Petromyzon_marinus |  |  | Sea lamprey |  | Fish |
| _1106_Salmo_salar |  |  | Atlantic salmon (freshwater only) |  | Fish |
| _1092_Austropotamobius_pallipes |  |  | White-clawed crayfish |  | Not included in a group |
| _1065_Euphydryas_aurinia |  |  | Marsh fritillary butterfly |  | Not included in a group |
| _1029_Margaritifera_margaritifera |  |  | Freshwater pearl mussel |  | Not included in a group |
| _1990_Margaritifera_durrovensis |  |  | Nore freshwater pearl mussel |  | Not included in a group |
| _1014_Vertigo_angustior |  |  | Narrow-mouthed whorl snail |  | Vertigo |
| _1013_Vertigo_geyeri |  |  | Geyer’s whorl snail |  | Vertigo |
| _1016_Vertigo_moulinsiana |  |  | Desmoulin’s whorl snail |  | Vertigo |
| _1364_Halichoerus_grypus |  |  | Grey seal |  | Marine mammal |
| _1355_Lutra_lutra |  |  | Otter |  | Not included in a group |
| _1365_Phoca_vitulina |  |  | Common seal |  | Marine mammal |
| _1351_Phocoena_phocoena |  |  | Harbour porpoise |  | Marine mammal |
| _1303_Rhinolophus_hipposideros |  |  | Lesser horseshoe bat (listed as water dependent for cave roosting sites only) |  | Not included in a group |
| _1349_Tursiops_truncatus |  |  | Bottle-nosed dolphin |  | Marine mammal |
| _1393_Drepanocladus_vernicos |  |  | Slender green feather-moss |  | Plant |
| _1833_Najas_flexilis |  |  | Slender Naiad |  | Plant |
| _1395_Petalophyllum_ralfsii |  |  | Petalwort (Liverwort) |  | Plant |
| _1528_Saxifraga_hirculus |  |  | Yellow marsh saxifrage |  | Plant |
| _1421_Trichomanes_speciosum |  |  | Killarney fern |  | Plant |

## APPENDIX 5:   Impacts and Activities influencing the conservation status of the site (adapted from Natura 2000 form).

The original list supplied by the European Commission DG Environment has been modified slightly by NPWS for use in conservation status assessment, with the recent addition of certain categories (in italics).

CODE	CATEGORY

Agriculture, Forestry

100	Cultivation

101		modification of cultivation practices

102		mowing/cutting

103		agricultural improvement

104		removal of limestone pavement

110	Use of pesticides

120	Fertilisation

130	Irrigation

140	Grazing

141		abandonment of pastoral systems

142		overgrazing by sheep

143		overgrazing by cattle

144		overgrazing by deer

145		overgrazing by goats

146		overgrazing by hares, rabbits, small mammals

147		overgrazing by geese

148		overgrazing, general

149		undergrazing

150	Restructuring agricultural land holding

151		removal of hedges and copses

152		removal of scrub

160	General Forestry management

161		forestry planting

162		artificial planting

163		forestry replanting

164		forestry clearance

165		removal of undergrowth

166		removal of dead and dying trees

167		exploitation without replanting

168		felling of native or mixed woodland

170	Animal breeding

171		stock feeding

180	Burning

190	Agriculture and forestry activities not referred to above

Fishing, hunting and collecting

200	Fish and Shellfish Aquaculture

210	Professional fishing

211		fixed location fishing

212		trawling

213		drift-net fishing

220	Leisure fishing

221		bait digging

230	Hunting

240	Taking/Removal of fauna, general

241		collection (insects, reptiles, amphibians.....)

242		taking from nest (falcons)

243		trapping, poisoning, poaching

244		other forms of taking fauna

250	Taking/Removal of flora, general

251                 		pillaging of floristic stations

290	Hunting, fishing or collecting activities not referred to above

Mining and Extraction of Minerals

300	Sand and gravel extraction

301		quarries

302		removal of beach materials

310	Peat Extraction

311		hand-cutting of peat

312		mechanical removal of peat

320	Exploration and extraction of oil or gas

330	Mines

331		open cast mining

332		underground mining

340	Salt Works

390	Mining and extraction activities not referred to above

Urbanisation, industrialisation and similar activities

400	Urbanised areas, human habitation

401		continuous urbanisation

402		discontinuous urbanisation

403		dispersed habitation

409		other patterns of habitation

410	Industrial or commercial areas

411		factory

412		industrial stockage

419		other industrial/commercial areas

420	Discharges

421		disposal of household waste

422		disposal of industrial waste

423		disposal of inert materials

424		other discharges

430	Agricultural structures

440	Storage of materials

490	Other urbanisation, industrial and similar activities

Transportation and communication

500	Communication networks

501		paths, tracks, cycling tracks

502		routes, autoroutes

503		railway lines, TGV

504		port areas

505		airport

506		aerodrome, heliport

507		bridge, viaduct

508		tunnel

509		other communications networks

510	Energy transport

511		electricity lines

512		pipe lines

513		other forms of energy transport

520	Shipping

530	Improved access to site

590	Other forms of transportation and communication

Leisure and Tourism

(some included under different headings)

600	Sport and leisure structures

601		golf course

602		skiing complex

603		stadium

604		circuit, track

605		hippodrome

606		attraction park

607		sports pitch

608		camping and caravans

609		other sport/leisure complexes

610	Interpretative centres

620	Outdoor sports and leisure activities

621		nautical sports

622		walking, horseriding and non-motorised vehicles

623		motorised vehicles

624		mountaineering, rock climbing, spieleology

625		gliding, delta plane, paragliding, ballooning

626		skiing, off-piste

629		other outdoor sports and leisure activities

690	Other leisure and tourism impacts not referred to above

Pollution and other human impacts/activities

700	Pollution

701		water pollution

702		air pollution

703		soil pollution

709		other forms or mixed forms of pollution

710	Noise nuisance

720	Trampling, overuse

730	Military Manoeuvres

740	Vandalism

790	Other pollution or human impacts/activities

Human induced changes in hydraulic conditions

(wetlands and marine environments)

800	Landfill, land reclamation and drying out, general

801		polderisation

802		reclamation of land from the sea, estuary or marsh

803		infilling of ditches, dykes, ponds, pools, marshes or pits

810	Drainage

811		management of aquatic and bank vegetation for drainage purposes

820	Removal of sediments (mud ...)

830	Canalisation

840	Flooding

850	Modification of hydrographic functioning, general

851		modification of marine currents

852		modifying structures of inland water course

853		management of water levels

860	Dumping, depositing of dredged deposits

870	Dykes, embankments, artificial beaches, general

871		sea defence or coastal protection works

890	Other human induced changes in hydraulic conditions

Natural processes (biotic and abiotic)

900	Erosion

910	Silting up

920	Drying out

930	Submersion

940	Natural catastrophes

941		inundation

942		avalanche

943		collapse of terrain, landslide

944		storm, cyclone

945		volcanic activity

946		earthquake

947		tidal wave

948		fire (natural)

949		other natural catastrophes

950	Biocœnotic evolution

951		accumulation of organic material

952		eutrophication

953		acidification

954		invasion by a species

960	Interspecific faunal relations

961		competition (example:  gull/tern)

962		parasitism

963		introduction of disease

964		genetic pollution

965		predation

966		antagonism arising from introduction of species

967		antagonism with domestic animals

969		other forms of mixed forms of interspecific faunal competition

970	Interspecific floral relations

971		competition

972		parasitism

973		introduction of disease

974		genetic pollution

975		lack of pollinating agents

976		damage by game species

979		other forms or mixed forms of interspecific floral competition

990	Other natural processes

## APPENDIX 6:   Natura 2000 sites for which Conservation Management Plans have been prepared

The public consultation process has been completed for the Natura 2000 sites listed below. The plans are available on the NPWS website at www.npws.ie

SAC 000115 - Ballintra

SAC 000116 - Ballyarr Wood

SAC 000129 - Croaghonagh Bog

SAC 000168 - Magheradrumman Bog

SAC 00018S - Sessiagh lough

SAC 000432 - Barrigone

SAC 000497 - Fughanv Bog

SAC 000S34 - Owenduff/Nephin Complex

SAC 000575 - Ferbane Bog

SAC 000576 - Fin Lough

SAC 000580 - Mongan Bog

SAC 000582 - Raheenmore Bog

SAC 000633 - Lough Hoe Bog

SAC 000634 - Lough Nabrlckkeagh

SAC 000636 - Templehouse & Cloonacleigha Loughs

SAC 000638 - Union Wood

SAC 000646 - Galtee Mountains

SAC 000647 - Kilcarren-Firville Bog

SAC 000770 - Blackstairs Mountains

SAC 000831 - Cullahill Mountain

SAC 000869 - Lisbigney Bog

SAC 000934 - Kilduff/Devllsblt Mountain

SAC 000939 - SiIvermine Mountains

SAC 001090 - Ballyness Bav

SAC 001107 - Coolvoy Bog

SAC 001125 - Dunragh Loughs & Pettigo Plateau

SAC 001179 - Muckish Mountain

SAC 00119S - Termon Strand

SAC 001197 - Keeper Hill

SAC 001656 - Bricklieve Mountains & Keishcorran

SAC 001669 - Knockalongy & Knockachree Cliffs

SAC 001680 - Streedagh Point Dunes

SAC 001766 - Magherabeg Dunes

SAC 001873 - Derryclogher (Knockaboy) Bog

SAC 001880 - Meenaguse Scragh

SAC 001881 - Maulagowna Bog

SAC 001912 - Glendree Bog

SAC 001975 - Ballyhoorlsky Point to Fanad Head

SAC 002117 - Lough Coy

SAC 002123 - Ardmore Head

SAC 002124 - Bolingbrook Hill

SAC 002164 - Lough Golagh & Breesy Hill

SPA 004017 - Mongan Bog

SPA 004098 - Owenduff/Nephin Complex

SPA 004099 - Dunragh Loughs Pettigo Plateau

## APPENDIX 7. RIVERS AND SACs IN WHICH MARGARITIFERA MARGARITIFERA IS CURRENTLY KNOWN TO OCCUR

The Tables below are reproduced from the Margaritifera margaritifera (the freshwater pearl mussel) conservation assessment Backing Document, prepared by Evelyn A. Moorkens, Ian J. Killeen and Eugene Ross, edited by Áine O Connor, July 2007. See end notes.

7.1. The Qualifying populations of Margaritifera margaritifera , for which Special Areas of Conservation (SACs) have been designated in Ireland. Sub-basin plans will apply to these populations under the Water Framework Directive.

|  | Freshwater pearl mussel population | SAC Site Code | SAC Site Name | Rivers and lakes containing Margaritifera (list not exhaustive) |
| --- | --- | --- | --- | --- |
| 1 | Bandon | 002171 | Bandon River cSAC | Bandon & Caha |
| 2 | Aughavaud (Barrow) | 002162 | River Barrow and River Nore cSAC | Aughavaud |
| 3 | Ballymurphy (Barrow) | 002162 | River Barrow and River Nore cSAC | Ballymurphy |
| 4 | Mountain (Barrow) | 002162 | River Barrow and River Nore cSAC | Mountain, Aughnabrisky |
| 5 | Bundorragha | 001932 | Mweelrea/ Shreefry/ Erriff Complex cSAC | Bundorragha |
| 6 | Caragh | 000365 | Killarney National Park, Macgillycuddy’s Reeks and Caragh River Catchment cSAC | Caragh, Owenroe, Meelagh, Caraghbeg, Glashawee, Lough Beg Stream, Lough Acoose, Cloon Lough |
| 7 | Clady | 000140 | Fawnboy Bog/ Lough Nacung cSAC | Clady |
| 8 | Corrib | 000297 | Lough Corrib cSAC | Owenriff, Glengawbeg |
| 9 | Currane | 000365 | Killarney National Park, Macgillycuddy’s Reeks and Caragh River Catchment cSAC | Capall, Cummeragh |
| 10 | Dawros | 002031 | The Twelve Bens/ Garraun Complex cSAC | Dawros |
| 11 | Eske | 000163 | Lough Eske and Ardnamona Wood cSAC | Eske |
| 12 | Kerry Blackwater | 002173 & 000365 | Blackwater River (Kerry) cSAC & Killarney National Park, Macgillycuddy’s Reeks and Caragh River Catchment cSAC | Blackwater, Kealduff, Derreendarragh |
| 13 | Gearhameen (Laune) | 000365 | Killarney National Park, Macgillycuddy’s Reeks and Caragh River Catchment cSAC | Gearhameen & Owenreagh |
| 14 | Glaskeelan (Leannan) | 002047 | Cloghernagore Bog and Glenveagh National Park cSAC | Glaskeelan |
| 15 | Leannan | 002176 | Leannan River cSAC | Leannan |
| 16 | Allow (Munster Blackwater) | 002170 | Blackwater River (Cork/Waterford) cSAC | Allow |
| 17 | Licky | 002170 | Blackwater River (Cork/Waterford) cSAC | Licky |
| 18 | Munster Blackwater | 002170 | Blackwater River (Cork/Waterford) cSAC | Munster Blackwater (main channel) |
| 19 | Newport | 002144 | Newport River cSAC | Newport |
| 20 | Nore | 002162 | River Barrow and River Nore cSAC | Nore |
| 21 | Owencarrow | 002047 | Cloghernagore Bog and Glenveagh National Park cSAC | Owencarrow |
| 22 | Owenea | 000197 | West of Ardara/Maas Road cSAC | Owenea |
| 23 | Owenmore | 000375 | Mount Brandon cSAC | Owenmore |
| 24 | Ownagappul | 001879 | Glanmore Bog cSAC | Ownagappul & Barrees |
| 25 | Shannon Estuary | 002165 | Lower River Shannon cSAC | Cloon |
| 26 | Slaney | 000781 | Slaney River Valley cSAC | Derreen |
| 27 | Suir | 002137 | Lower River Suir cSAC | Clodiagh |

7.2. ‘Non-qualifying’ populations of Margaritifera margaritifera in Special Areas of Conservation (SACs).

Note that these populations were not included in the original listing of Qualifying Interests for individual SACs. Surveys to clarify the extent and status of these populations are in progress. This species is protected under the Wildlife Acts.

| SAC Site Code | Name of SAC | County | Rivers / Lakes |
| --- | --- | --- | --- |
| 000365 | KILLARNEY NATIONAL PARK, MACGILLYCUDDY'S REEKS AND CARAGH RIVER CATCHMENT | Kerry | Finnihy, Cottoners, Flesk, Lough Leane, Finnow, Owneykeagh, |
| 001932 | MWEELREA/SHEEFFRY/ERRIFF COMPLEX | Mayo | Erriff |
| 002165 | LOWER RIVER SHANNON | Kerry | Feale |
| 00090 | GLENGARRIFF HARBOUR AND WODDLAND SAC | Cork | Glengarriff |
| 0002298 | RIVER MOY | Mayo | Deel |
| 002170 | BLACKWATER RIVER (CORK/WATERFORD) | Cork/ Waterford | Owentaraglin |
| 000375 | MOUNT BRANDON | Kerry | Owenascaul |
| 000781 | SLANEY RIVER VALLEY | Wicklow/ Carlow | Slaney, Derry, Bann |
| 002137 | LOWER RIVER SUIR | Tipperary/ Waterford | Clodiagh (Tipp), Aherlow, Multeen |
| 000163 | LOUGH ESKE AND ARDNAMONA WOOD | Donegal | Lough Eske |

7. 3.  Rivers with Margaritifera margaritifera populations with no existing conservation designation.

Surveys to clarify the extent and status of these populations are in progress. This species is protected under the Wildlife Acts.

| River | Catchment | County | River | Catchment | County |
| --- | --- | --- | --- | --- | --- |
| Tar | Suir | Waterford | Laune | Laune | Kerry |
| Tay | Tay | Waterford | Lough Currane | Currane | Kerry |
| Mahon | Mahon | Waterford |  |  |  |
| Owneykeagh | Flesk | Kerry | Owreagh | Owreagh | Kerry |
| Lee | Lee | Cork | Brown Flesk | Maine | Kerry |
| Sullane | Lee | Cork | Kilcurry | Castletown | Louth |
| Foherish | Sullane Lee | Cork | Galey | Feale | Kerry |
| Laney | Sullane Lee | Cork | Sheen | Sheen | Kerry |
| Toon | Lee | Cork | Baurearagh | Sheen | Kerry |
| Mealagh | Mealagh | Cork | Roughty | Roughty | Kerry |
| Owenaher | Moy | Sligo | Owenshagh | Owenshagh | Kerry |
| Easky | Easky | Sligo | Glanrastel | Owenshagh | Kerry |
| Owenacahina | Glengarriff | Cork | Glantrasna | Owenshagh | Kerry |
| Coomerkane | Glengarriff | Cork | Dromoghty | Dromoghty | Kerry |
| Ilen | Ilen | Cork | Bunnow | Bunnow | Kerry |
| Saivnose | Ilen | Cork | King’s | Liffey | Wicklow |
| Owennashingaun | Ilen | Cork | Avoca | Avoca | Wicklow |
| Glan (Dunmanus) | Glan | Cork | Aughrim | Avoca | Wicklow |
| Roury | Roury | Cork | Derry Water | Aughrim Avoca | Wicklow |
| Barrow | Barrow | Carlow | Ow | Aughrim Avoca | Wicklow |
|  |  |  | Avonmore | Avoca | Wicklow |
| Blackwater | Bandon | Cork | Vartry | Vartry | Wicklow |
| Coomhola | Coomhola | Cork | Tobergal | L. Cullin | Mayo |
| Owvane | Owvane | Cork | Crumpaun | Newport | Mayo |
| Leamawaddra | Leamawaddra | Cork | Owenwee | Owenwee | Mayo |
| Four mile water | Four mile water | Cork | Bunowen | Bunowen | Mayo |
| Trafrask | Trafrask | Cork | Finny | Lough Mask | Mayo |
| Adrigole | Adrigole | Cork | Feenone | Roonah Lough | Mayo |
| Clashduff | Adrigole | Cork | Graney | Scarriff, L Derg | Clare |
| Glashaboy | Glashaboy | Cork | Erne | Erne | Cavan |
| Derryneen | Recess | Galway | Annalee | Erne | Cavan |
| Owentooey | Recess | Galway | Swanlinbar | Erne | Cavan |
| Ballynahinch Lake | Recess | Galway | Larah | Erne | Cavan |
| Recess | Recess | Galway | Liffey | Liffey | Dublin |
| Lough Inagh | Recess | Galway | Lough Fern | Leannan | Donegal |
| Furbo (Knock) | Lough Inch system | Galway |  |  |  |
| Glengawbeg | Owenriff Corrib | Galway | Finn | Foyle | Donegal |
| Woodford | Woodford L. Derg | Galway | Derg | Lougn Derg | Donegal |
| Tullaghobegly | Tullaghobegly | Donegal | Carrownamaddy | Carrownamaddy | Donegal |
| Waterfoot | Erne | Donegal | Eany | Eany water | Donegal |
| Stragar | Stragar | Donegal | Oily | Oily | Donegal |
| Loughaderry | Stragar | Donegal |  |  |  |

## APPENDIX 8.   SACS IN WHICH WATER DEPENDENT HABITATS AND/OR SPECIES ARE ASSESSED AS BEING AT UNFAVOURABLE CONSERVATION STATUS (2007/08 reporting).

| SAC at Unfavourable Conservation Status |  |  |  |
| --- | --- | --- | --- |
| SITECODE | SITE_NAME | UCS_Issue | Water_Body_at_UCS |
| 000007 | LOUGH OUGHTER AND ASSOCIATED LOUGHS SAC | Eutrophication of lakes from diffuse and point sources. Freshwater pearl mussel population in Erne River | Lough Oughter SEG_CD 36_675, and associated water bodies in Lough Oughter Complex SAC. See M. margaritifera database |
| 000014 | BALLYALLIA LAKE SAC | Eutrophication of lake habitat | 27_72, 27_243, 27_104 |
| 000051 | LOUGH GASH TURLOUGH SAC | Eutrophication of turlough, Newmarket on Fergus WWTP discharge to turlough a likely factor | 27_116, ground water body SH_G_121 |
| 000054 | MONEEN MOUNTAIN SAC | Coastal Lagoon Muckinish lake | 29_131 / TWB_WE_110_0100 |
| 000090 | GLENGARRIFF HARBOUR AND WOODLAND SAC | Freshwater pearl mussel population in Coomerkame and Glengarriff Rivers | See M. margaritifera database |
| 000091 | CLONAKILTY BAY SAC | Coastal Lagoons Clogheen/White's Marsh and Inchydoney highly eutrophic and with hydrographic impacts | 20_8 / CWB_SW_100_0200 and 20_9 / CWB_SW_100_0300 |
| 000093 | CAHA MOUNTAINS SAC | Headwaters of several M. margaritifara rivers rise in this SAC, the species may also be present. | See M. margaritifera database |
| 000138 | DURNESH LOUGH SAC | Coastal Lagoon Durnesh Lough highly eutrophic | 37_207 / TWB_NW_040_0100 |
| 000140 | FAWNBOY BOG/LOUGH NACUNG SAC | Freshwater pearl mussel qualifying population in Clady River | See M. margaritifera database |
| 000163 | LOUGH ESKE AND ARDNAMONA WOOD SAC | Freshwater pearl mussel qualifying population in Eske River | See M. margaritifera database |
| 000185 | SESSIAGH LOUGH SAC | Eutrophication noted as a pressure for Arctic char, potential future impact on Najas flexilis | Sessiagh Lough 38_61 |
| 000197 | WEST OF ARDARA/MAAS ROAD SAC | Freshwater pearl mussel qualifying population in Owenea River. Najas flexilis in Clooney Lough. | See M. margaritifera database. Lake 38_542 |
| 000205 | MALAHIDE ESTUARY SAC | Broadmeadow Lagoon moderately eutrophic | TWB_EA_060_0100 |
| 000206 | NORTH DUBLIN BAY SAC | Golf course ground water abstraction impacting on dune slack habitat, North Bull Island | Water body local to dune slack |
| 000212 | INISHMAAN ISLAND SAC | Possible drying out of machair habitat. Lagoon Port na Cora moderately eutrophic | TWB_WE_030_0100 |
| 000213 | INISHMORE ISLAND SAC | Lagoon Loch an tSaile mildly eutrophic. Lagoons Phort Chorruch, L an Chara and L. Dearg moderately eutrophic | TWB_WE_050_0100. TWB_WE_070_0100; 060_0100; 080_0100 |
| 000268 | GALWAY BAY COMPLEX SAC | Lagoons Murree, Aughinish. Rossalia, Bridge Lough, Doorus lagoons and Tureen lagoon moderately eutrophic. Rincarna Pools and L. Atalia highly eutrophic | See Coastal Lagoons database |
| 000278 | INISHBOFIN AND INISHSHARK SAC | Lagoon L. Bofin | TWB_WE_280_0100 |
| 000297 | LOUGH CORRIB SAC | Freshwater pearl mussel qualifying population in Owenriff River. Lough Corrib: Najas status, eutrophication, alien species | Owenriff river, see M. margaritifera database. Lake 30_666b, 30_666a |
| 000332 | AKERAGH, BANNA AND BARROW HARBOUR SAC | Eutrophication of dune slacks and saltmarsh |  |
| 000343 | CASTLEMAINE HARBOUR SAC | Freshwater pearl mussel population in Cottoner's river | See M. margaritifera database |
| 000365 | KILLARNEY NATIONAL PARK, MACGILLYCUDDY'S REEKS AND CARAGH RIVER CATCHMENT SAC | Freshwater pearl mussel Q populations in Caragh, Caraghbeg, Meelagh, Owenroe, Loughs Caragh and Acoose; P in Cappal, Finnihy, Cottoners, Flesk, Finnow, Owneykeagh, Owenreagh Rivers, and ; extinct in L. Leane. Najas flexilis UCS in L. Adoolig, Upper L, Muckross, L | See M. margaritifera and Najas flexilis databases |
| 000375 | MOUNT BRANDON SAC | Freshwater pearl mussel qualifying population in Owenmore River, population in Owenascaul River | See M. margaritifera database |
| 000396 | POLLARDSTOWN FEN SAC | Vertigo geyeri, angustior and moulinsiana; ground water dependent cladium fen, alkaline fen and petrifying spring; adverse hydrological impacts including abstraction | Ground water body IESEG106, National Abstractions Register PT_CD SE_ABS0420 at SAC boundary, other abstractions may be relevant also |
| 000397 | RED BOG, KILDARE SAC | Ground water abstraction impacting on SAC habitats, adjoining quarry to south of site may also affect hydrology | Ground water body IE_EA_G_003; National Abstractions Register ORG_PT_CD 1400PUB1024 |
| 000571 | CHARLEVILLE WOOD SAC | Vertigo moulinsiana, ongoing monitoring and management of water levels by NPWS required | Ground water body IE_SH_G_103 and possibly others |
| 000576 | FIN LOUGH (OFFALY) SAC | Vertigo geyeri, drainage of bog, drain blocking required by NPWS management plan. GW abstraction nearby may also be an issue. Vertigo moulinsiana habitat transitional | National Abstractions Register SH_ABS0092? |
| 000607 | ERRIT LOUGH SAC | Water abstraction | 26_702; GWB_CD IE_SH_G_072; PT_CD SH_ABS0166 |
| 000622 | BALLYSADARE BAY SAC | Coastal Lagoon Tanrego highly eutrophic | 35_216 / TWB_WE_460_0400 |
| 000627 | CUMMEEN STRAND/DRUMCLIFF BAY (SLIGO BAY) SAC | Vertigo angustior, impacted by dune slack drying. Water abstraction? | Several ground water bodies? |
| 000633 | LOUGH HOE BOG SAC | Lough Talt Arctic char spawning and nursery habiitat adversely impacted by abstraction. Vertigo geyeri, evidence of unfavourable status in 2008 survey, in flush habitat at L. Talt, possible hydrological pressures | Lough Talt 34_405, National Abstractions Register WE_ABS0024 |
| 000688 | LOUGH OWEL SAC | Eutrophication | 26_703 |
| 000704 | LADY'S ISLAND LAKE SAC | Coastal Lagoon habitat Lady's Island Lake highly eutrophic, hydrological pressures | 13_34/TWB SE_060_0100 |
| 000708 | SCREEN HILLS SAC | Eutrophication of groundwater | GWB Ms_cd IESEGG084 |
| 000709 | TACUMSHIN LAKE SAC | Coastal lagoon Tacumshin: drainage/hydrographic pressures on lagoon habitat | 13_35/TWB SE_070_0100 |
| 000729 | BUCKRONEY-BRITTAS DUNES AND FEN SAC | Dune slack habitats drying due to water abstraction and hydromorphological pressures. Fen habitat may also be affected. | Local area within ground water body Ms_cd IE_EA_G_004? National Abstractions Register, 3400Private111, 3400PUB1050, 3400PUB1045 + unregistered surface water abstraction for golf course |
| 000781 | SLANEY RIVER VALLEY SAC | Freshwater pearl mussel qualifying population in Dereen river, other populations in Slaney, Derry and Bann. Status of allis and twaite shad fish populations in the Slaney uncertain. | See M. margaritifera database |
| 000859 | CLONASLEE ESKERS AND DERRY BOG SAC | Vertigo geyeri, possible land drainage and water abstraction pressures | Ground water body IE_SH_G_103, IE_SH_G_066 underlie SAC |
| 000869 | LISBIGNEY BOG SAC | Apparent loss of Vertigo moulinsiana through drainage |  |
| 001021 | CARROWMORE POINT TO SPANISH POINT AND ISLANDS SAC | Coastal Lagoon Lough Donnell moderately eutrophic, hydromorphology - barrier collapse | 28_78 / TWB_SH_090_0100 |
| 001040 | BARLEY COVE TO BALLYRISODE POINT SAC | Coastal Lagoon Lisagriffin lake highly eutrophic | TWB_SW_140_0200 |
| 001058 | GREAT ISLAND CHANNEL SAC | Coastal lagoon Ballyvodock lagoon moderately eutrophic | 19_123 |
| 001061 | KILKERAN LAKE AND CASTLEFREKE DUNES SAC | Coastal Lagoon Kilkieran lake highly eutrophic, hydrographic pressure | 20_160/TWB SW_110_0100 |
| 001090 | BALLYNESS BAY SAC | Vertigo geyeri |  |
| 001141 | GWEEDORE BAY AND ISLANDS SAC | Najas flexilis in Mullaghderg Lough(s), eutrophication cited. Najas in Lough Ibby extinct, disposal of household waste cited. Coastal Lagoon Kincas Lough moderately eutrophic | Mullaghderg Lough 38_81 and 38_85. Lough Ibby 38_579. Kincaslough 38_689 / TWB_NW_160_0100 |
| 001342 | CLOONEE AND INCHIQUIN LOUGHS, URAGH WOOD SAC | Najas flexilis status unceratin in Cloonee Lough | 21_446 |
| 001387 | BALLYNAFAGH LAKE SAC | Vertigo moulinsiana |  |
| 001398 | RYE WATER VALLEY/CARTON SAC | Vertigo angustior, Vertigo moulinsiana, habitat at Louisa Bridge drying out, hydrological pressure from urban development? |  |
| 001482 | CLEW BAY COMPLEX SAC | Coastal Lagoon Claggan lagoon moderately eutrophic. Vertigo geyeri continued monitoring required | Water body requires mapping and assignment of code |
| 001513 | KEEL MACHAIR/MENAUN CLIFFS SAC | Najas flexilis close to extinction in Keel Lough, eutrophication | Keel Lough 33_1895 |
| 001529 | LOUGH CAHASY, LOUGH BAUN AND ROONAH LOUGH SAC | Coastal lagoon Roonagh lagoon moderately eutrophic | TWB_WE_330_0100 |
| 001626 | ANNAGHMORE LOUGH (ROSCOMMON) SAC | Vertigo geyeri monitoring should include review of water abstraction from Lough Nablasbarnagh | Lough Nablasbarnagh 26_277, National Abstractions Register SH_ABS0006 |
| 001673 | LOUGH ARROW SAC | Eutrophication | Lough Arrow 35_159 |
| 001741 | KILMUCKRIDGE-TINNABERNA SANDHILLS SAC | Eutrophication |  |
| 001774 | LOUGH CARRA/MASK COMPLEX SAC | Eutrophication in lake. Freshwater mussel population in River Finny | 30_665 and 30_347. See M. margaritifera database |
| 001879 | GLANMORE BOG SAC | Freshwater pearl mussel qualifying population in Owenagappul River and Barrees Stream | See M. margaritifera database |
| 001898 | UNSHIN RIVER SAC | Requires Q values of 4.5 to 5 to ensure favourable conservation status | River Unshin |
| 001919 | GLENADE LOUGH SAC | Najas status uncertain | 35_156 |
| 001922 | BELLACORICK BOG COMPLEX SAC | Vertigo geyeri, hydrological change/drainage or other habitat change, drain blocking required by NPWS management plan |  |
| 001932 | MWEELREA/SHEEFFRY/ERRIFF COMPLEX SAC | Freshwater pearl mussel qualifying population in Bundorragha; population in Erriff River. Najas status uncertain in Fin Lough. Coastal Lagoon Corragaun Lough moderately eutrophic. Vertigo geyeri | See M. margaritifera database. Fin Lough 32_391. Corragaun Lough 32_443 / TWB_WE_320_0100. Vertigo geyeri requires grazing management |
| 001975 | BALLYHOORISKY POINT TO FANAD HEAD SAC | Vertigo angustior, water abstraction? |  |
| 002008 | MAUMTURK MOUNTAINS SAC | Freshwater pearl mussel population in Recess catchment | See M. margaritifera database |
| 002031 | THE TWELVE BENS/GARRAUN COMPLEX SAC | Freshwater pearl mussel qualifying population in Dawros. Part of population in Recess River catchments? | See M. margaritifera database |
| 002034 | CONNEMARA BOG COMPLEX SAC | Najas flexilis status uncertain in lakes Anillanaucht/Anillaunlughy, Nalawney and Bollard. Freshwater pearl mussel population in Recess catchment | 31_169, 31_216, 31-35. See M. margaritifera database |
| 002047 | CLOGHERNAGORE BOG AND GLENVEAGH NATIONAL PARK SAC | Freshwater pearl mussel qualifying populations in Owencarrow and Glaskeelan Rivers | See M. margaritifera database |
| 002070 | TRALEE BAY AND MAGHAREES PENINSULA, WEST TO CLOGHANE SAC | Water abstraction. Coastal Lagoons Blennerville lake highly eutrophic, Lough Gill moderately eutrophic with hydrographic pressures. Freshwater pearl mussel qualifying population in Owenmore River | See M. margaritifera database. Coastal lagoons Lakes 23_72 / TWB SH_040_0100, and TWB SH_050_0100. GWBs SH_G_044, _008, _228 +? |
| 002074 | SLYNE HEAD PENINSULA SAC | Coastal Lagoon L. Ballyconneely moderately eutrophic | 31_162 / TWB_WE_240_0100 |
| 002111 | KILKIERAN BAY AND ISLANDS SAC | Coastal Lagoons Loch Fhada upper pools and L. Ateesky | TWB_WE_200_0300 and 31_1064 |
| 002120 | LOUGH BANE AND LOUGH GLASS SAC | Water abstraction Lough Bane | Lough Bane SEG_CD 07_270, National Abstractions Register PT_CD EA_ABS0010 and ORG_PT_ CD_SCH 2300PUB1011 |
| 002137 | LOWER RIVER SUIR SAC | Freshwater pearl mussel qualifying populations in River Clodiagh (Portlaw), and populations in Agherlow, Clodiagh, Multeen, Tar. | See M. margaritifera database |
| 002141 | MOUNTMELLICK SAC | Vertigo moulinsiana, monitoring and management of water levels by NPWS required in future to prevent drying out of habitat in old canal | water body not mapped by EPA |
| 002144 | NEWPORT RIVER SAC | Freshwater pearl mussel qualifying population in Newport River | See M. margaritifera database |
| 002147 | LISDUFF FEN SAC | Vertigo geyeri, Vertigo moulinsiana, may be hydrological issues, drainage/abstraction? |  |
| 002158 | KENMARE RIVER SAC | Coastal Lagoon Lauragh. Vertigo angustior | Lauragh TWB_SW_190_0200 |
| 002162 | RIVER BARROW AND RIVER NORE SAC | Freshwater pearl mussel qualifying population in Aughnabrisky, Ballymurphy, Mountain Rivers, Irish freshwater pearl mussel in Nore only. Vertigo moulinsiana | See M. margaritifera database and M. durrovensis database |
| 002165 | LOWER RIVER SHANNON SAC | Freshwater pearl mussel qualifying population in Cloon River, population in Feale. Coastal Lagoons Cloonconeen Pool, Scattery Island, Quayfield and Shannon Airport | Rivers Cloon and Feale, see M. margaritifera database. Lagoons Cloonconeen SEG_CD 27_40, Scattery Island CWB SH_060_1300, Quayfield SEG_CD 24_81, Shannon Airport TWB SH_060_1000 |
| 002170 | BLACKWATER RIVER (CORK/WATERFORD) SAC | Freshwater pearl mussel qualifying populations in Licky, Blackwater, Allow Rivers, population in Owentaraglin | See M. margaritifera database |
| 002171 | BANDON RIVER SAC | Freshwater pearl mussel qualifying population in Bandon and Caha Rivers | See M. margaritifera database |
| 002173 | BLACKWATER RIVER (KERRY) SAC | Freshwater pearl mussel qualifying population in Blackwater, Dereendaragh, Kealduff Rivers | See M. margaritifera database |
| 002176 | LEANNAN RIVER SAC | Freshwater pearl mussel qualifying population in Leannan River | See M. margaritifera database |
| 002249 | THE MURROUGH WETLANDS SAC | Coastal lagoon Kilcoole lagoon moderately eutrophic | TWB_EA_120_0100 |
| 002259 | TORY ISLAND COAST SAC | Coastal Lagoon Lough O Dheas moderately eutrophic | TWB_NW_180_0100 |
| 002283 | RUTLAND ISLAND AND SOUND SAC | Coastal Lagoon Sally's Lough | SEG_CD 38_676 / CWB_NW_150_0100 |
| 002287 | LOUGH SWILLY SAC | Coastal Lagoons Blanket Nook Lough and Inch Lough moderately eutrophic | SEG_CD 39_59 / TWB_NW_220_0200 and 39_14 / TWB_NW_220_0300 |
| 002298 | RIVER MOY SAC | Freshwater pearl mussel populations in Deel, Tobergal, Owenaher, and Moy | See M. margaritifera database |
| 002301 | RIVER FINN SAC | Freshwater pearl mussel population in River Finn | See M. margaritifera database |

Note that databases are in preparation for the 27 populations of Margaritifera margaritifera that will be subject to sub-basin plans under the Water Framework Directive. Refer to Habitats Directive WFD measures 7.3.1 to 8. Refer to Appendix 7 for currently available distribution information for M. margaritifera

## APPENDIX 9: NOTES ON COASTAL LAGOONS DATABASE

COASTAL LAGOONS: EU Code 1150; priority Annex 1 listed habitat (Habitats Directive 92/43/EEC)

Water dependent habitat

WATER SOURCE:

Transitional

Coastal

Surface

Ground

HABITAT DEFINITION:

"Expanses of shallow coastal salt water, of varying salinity or water volume, wholly or partially separated from the sea by sand banks or shingle, or, less frequently, by rocks. Salinity may vary from brackish water to hypersalinity depending on rainfall, evaporation and through the addition of fresh seawater from storms, temporary flooding by the sea in winter or tidal exchange. With or without vegetation from Ruppietea maritimae, Potametea, Zosteretea or Charetea." - CORINE 91:23.21 OR 23.22

Coastal lagoons are..."areas of typically (but not exclusively) shallow, coastal saline water" which "retain a proportion of their water at low tide and may develop as brackish, fully saline or hypersaline water bodies." - Bamber et al. (2001).

"As landforms, coastal lagoons are highly dynamic and are transitional between open coast, tidal estuary and freshwater lake. Within a lagoon, environmental conditions (in particular salinity but also such factors as pH, temperature and turbidity) can vary significantly both spatially and temporally.." - Oliver (2007).A key feature of all lagoons is restricted tidal exchange.

WATER QUALITY/QUANTITY ANTHROPOGENIC PRESSURES:

1. Eutrophication. Excessive nutrient inputs, principally due to un-sewered dwellings, agriculture, and waste-water treatment plant effluent, leading to algal blooms and fish kills in some cases, and affecting over 61% of the overall area of lagoon habitat in Ireland.

Conservation trend: Unfavourable-BAD

2. Drainage and modification of hydrographic functioning, including modification of marine currents, modifying structures of inland water course, and management of water levels, with the aim of reducing lagoon flooding. Use of non-return valves is increasing and is tending to reduce lagoon salinity.

Conservation trend: Unfavourable-inadequate.

3. Other water quality/quantity anthropogenic pressures are less widespread, and include accumulation of organic material, industrial/ commercial activities, aquaculture, golf course development, un-sewered/inadequately treated effluent from camping or caravan parks, invasion by exotic species, excessive poaching by cattle, dumping, removal of beach material, and landfill and land reclamation.

WATER QUALITY/QUANTITY NATURAL PRESSURES:

Erosion, silting up/drying out, accumulation of organic material e.g. algal material of lagoon origin, or of marine origin following overtopping of the barrier during storms and on-shore winds), sea level rise.

ECOLOGICAL REQUIREMENTS:

Most lagoons naturally occur in the upper oligo to lower eutrophic category. Shock lagoons can occur where decaying seaweed causes anoxic conditions, hypertrophic conditions can also occur and are not well understood, but are likely to be temporary and thus unlike anthropogenic pressures.

Nutrients, hydrochemistry and hydrography in lagoons are site and water body-specific, and are not well understood (Curtis et al (2006) includes a number of recommendations in this regard). However, there is clear evidence of damage to lagoon habitat arising from eutrophication and morphological change in particular, and remedial action is not precluded by data limitations.

DATABASE INFORMATION:

Point locations for coastal lagoons were provided by NPWS, and were cross-referenced to conservation designations (SAC, SPA, pNHA). One lagoon site has been added to the database in consultation with Dr Geoff Oliver: Renmore Lough in Galway City,TW body WE_170_0600.

Lagoon point locations were cross-referenced to lake water bodies (field SWB_SEG_CD), transitional water bodies (field TWB_Ms_cd), and coastal water bodies (field CWB_Ms_cd). A number of point location errors were identified, and the point locations and grid references were amended in consultation with Dr Geoff Oliver to ensure that lagoons are identified to the correct water body. The correct grid references are given in the Val_x and Val-Y fields; the original grid references supplied by NPWS are given in the Old_Northing and Old_Easting fields. The 'Grid_ref_amended' field indicates whether a grid reference was amended or not for a point which was considered doubtful. Some lagoon points are shown as centroids between multiple water bodies (e.g. North Slob, Co. Wexford) - these instances are noted in the comments on water body layers field. This field also notes instances where there are errors in the surface water body, transitional wb and coastal wb tables, i.e. where a water body is assigned to two categories, or is not mapped. As lagoons are priority Annex 1 listed water dependent habitats in which restricted tidal range is a key feature irrespective of salinity range, it is recommended that all known sites are included as transitional water bodies.No edits have been made to the water body map layers.

The Area_ha field gives the lagoon size; these data were taken from the Conservation Status Assessment Report Coastal Lagoons (1150), March 2007, Appendix 1, except for Tacumshin where the recent (since 2000) recorded maximum area of the lagoon of 257ha has been taken as the lagoon area (Conservation Status Assessment Report page 36).

Salinity range (psu) data are taken from table 4.3.1. in Oliver 2005.

The Morphological_type field data are taken from Table 2 and Appendix 1 of the Conservation Status Assessment Report. Sedimentary lagoon indicated with an asterisk are those separated from the sea by a cobble barrier.

The Biological_classification field data are taken from Table 4.1.1 of Oliver 2005. Biological classification data are not available for all lagoons.

The Conservation_status field data are taken from Appendix 2 of the Conservation Status Assessment Report, and use the four categories used for reporting on Annexed habitats under the Habitats Directive: Favourable, Unfavourable-inadequate, Unfavourable-BAD, and Unknown.

The Trophic_status field data are taken from Figure 5 of the Conservation Status Assessment Report, with reference also to Appendix 2. The 3 categories of mildly, moderately and highly eutrophic are used, and refer solely to eutrophication due to anthropogenic sources i.e. septic tanks/un-sewered properties, agriculture, and waste-water treatment effluent.

The Hydrographic_drainage_pressure field data are taken from Appendix 3 of the Conservation Status Assessment Report, and refer to individual lagoons where adverse impacts are noted as a result of human intervention.

REFERENCES.

1.   Oliver (2007). Conservation Status Assessment Report: Coastal Lagoons (1150). March 2007. Dr Geoff Oliver, Cape Clear, Skibbereen, Co. Cork. Report to NPWS.

2.   Oliver (2005). Seasonal changes and biological classification of Irish coastal lagoons. G. A. Oliver,Ph.D. Thesis 2005, Department of Zoology, University College Dublin.

3.   Bamber, R.N, Gilliland, P.M. and Shardlow, M.E.A. (2001). Saline lagoons: a guide to their management and creation (interim version). ISBN 1 85716573 X Peterborough, English Nature.

4.   Curtis, Tom, Sheila Downes and Bernadette Ni Chathain (2006). The Water Framework Directive. North-South Share Project: Register of Protected Areas. Report on the Ecological Requirements of Water Dependent Habitats and Species designated under the Habitats Directive (Draft). RPS Consulting Engineers.

INFORMATION COMPILED BY: ELEANOR MAYES, WESTERN RBD. Updated 8.09.2008.

## APPENDIX 10.   Definition Table: Najas flexilis lakes DATABASE

Fields 19

EPA_LAKE_NAME

SEG_CD

LAKE_AREA

OTHER_NAME

POPULATION_SIZE:   C= 1-5 plants, D= very large; E= extinct,                                                        L= large; S= small; ? uncertain

CONSERVATION_STATUS; determined in consultation with Cilian Roden

GRID_LETTER

EASTING (Float)

NORTHING (Float)

COUNTY

DATE_FIRST_RECORDED

DATE_LAST_RECORDED

DATE_LAST_SEARCHED

NUTRIENT_DATA_AVAILABLE

LOWER_MACROPHYTE_DEPTH ; B= benthic, no lower limit to vegetation

SAC_SITECODE

pNHA_SITECODE

NPWS_SITENAME

GRID_REF_AMENDED_WRBD

The Annex II listed plant slender naiad Najas flexilis occurs in lakes that are transitional between hard water and soft water types. It has been recorded in a total of 49 lakes, and continues to be present in 46 lakes, with three known extinctions, and a number of sites at which its status in uncertain. Najas is a small annual plant, and population size can vary substantially between years; because of this uncertain status can not be taken as proof of extinction currently, but indicates that further surveys should be informed by assessment of risks and pressures. Conservation status is assessed as favourable in 29 lakes, 8 of which are not covered by a nature conservation designation currently. The species occurs in Donegal, Leitrim, Mayo, Galway and Kerry. Connemara in west Galway appears to be the species Irish stronghold with 25 of the 49 recorded populations occurring there. Eutrophication from agriculture and dispersed habitation are the main pressures/threats; acidification is also a threat. The full site list and details for Najas flexilis is held by NPWS.

## APPENDIX 11.   Definition Table: Arctic char DATABASE

Data provided by Irish Char Conservation Group in June 2008. Refer to Fran Igoe, Shannon Regional Fisheries Board with any queries.

Database updated by WRBD with EPA lake SEG_CD codes, NPWS conservation site designations, and National Abstractions Register codes

Type NATIVE Charset "WindowsLatin1"

Fields 20

Lake_name

Lake_name_1;

County

SEG_CD : identifying EPA Code for the lake

Hydrometric_area

Area_ha

Char_status:     P =present, SU = status unknown

Afforestation:    Adverse impacts where noted by ICCG are ranked 1=slight; 2= moderate; 3 =severe.

Eutrophication: Adverse impacts where noted by ICCG are ranked as above

Overgrazing: Adverse impacts where noted by ICCG are ranked as above                Water_abstraction: Adverse impacts where noted by ICCG are ranked as above

Water_abstractions_register: Code for abstraction point

Other impacts noted

SAC_SITECODE Char (15) ;

pNHA_SITECODE Char (15) ;

NPWS_SITEMAME Char (50) ;

Fisheries management data provided by ICCG have not been included in this database.

Lakes listed as extinct by ICCG have not been included in this database. Note Minitiaghs Lough in Donegal is not included in the database, it was indicated as SU by ICCG, but listed as extinct in previous publications.

## APPENDIX 12.   WFD MEASURES UNDER THE HABITATS DIRECTIVE, AND FOR HIGH STATUS SITES

| Habitats Directive Measure | Action | Objective of measure | Responsible Agency | Type | GIS Locatable | Appropriate Assessment Required | High Status Sites Measure |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Identification of reasons for unfavourable conservation status |  |  |  |  |  |  |
|  | Measures by NPWS to clarify whether water status is an issue |  |  |  |  |  |  |
| HD7111 | Continued monitoring and survey by NPWS to confirm conservation status | Protect/Restore | NPWS | Monitoring | Y |  |  |
| HD7112 | Clarification of location of the features at unfavourable conservation status | Restore | NPWS | Monitoring | Y |  |  |
| HD7113 | Implementation of land management measures within protected areas | Protect/Restore | NPWS | Management | Y |  | HS 1 |
| HD7114 | Notification of appropriate agencies if water status issues are found to arise | Restore | NPWS | Management | N |  |  |
|  | Measures by River Basin Districts and implementing agencies to clarify how water status contributes to unfavourable conservation status |  |  |  |  |  |  |
| HD7121 | Update National Abstractions Register | Protect | EPA | Data | Y |  |  |
| HD7122 | Identify alternative solutions where existing abstractions are implicated in causing unfavourable conservation status | Restore | Local Authorities/ Private / EPA | Management | N | Y |  |
| Habitats Directive Measure | Action | Objective of measure | Responsible Agency | Type | GIS Locatable | Appropriate Assessment Required | High Status Sites Measure |
| HD7123 | Use existing analyses of risks and pressures in relevant catchments to identify the suite of WFD Measures that will contribute to resolving unfavourable conservation status for individual SACs. Include examination of ortho-photography in the review. LA Biodiversity Officers should be involved. | Restore | Local Authorities | Management | N | Y |  |
|  | Remedial Measures |  |  |  |  |  |  |
| HD721 | Consult with results of Measure 7.1.2 above, identify additional risks and sensitivities, and any additional Measures relevant to individual sites/features at unfavourable conservation status, including investigative/operational monitoring needs | Restore | Local Authorities / NPWS | Management | N | Y |  |
| HD722 | Document plans specific to each SAC unfavourable conservation status issue, (with reference to Sections 4 and 5 of the guidance, submit to NPWS for approval, and implement | Restore | Local Authorities / NPWS | Management | Y | Y |  |
|  | Specific measures |  |  |  |  |  |  |
|  | Measures for Margaritifera species (in addition to sub-basin plans) |  |  |  |  |  |  |
| HD7311 | GIS mapping of the currently known extent of currently non-qualifying M. margaritifera populations onto the EPA river sections database, including any distribution updates available from the rapid assessment survey during 2008, with annual updates in 2009 and 2010 with any additional distribution data – this database simply records known presence of the species. | Protect | NPWS | Data | Y |  |  |
| Habitats Directive Measure | Action | Objective of measure | Responsible Agency | Type | GIS Locatable | Appropriate Assessment Required | High Status Sites Measure |
| HD7312 | Notification and distribution by NPWS of the GIS database to all relevant Local Authorities and other agencies including the EPA, DAFF, Forest Service, Coillte, COFORD, Central Fisheries Board and Regional Fisheries Boards, so that Margaritifera populations can be taken into account as sensitive receptors in the development of River Basin Management Plans, and in the consideration of future plans and projects | Protect | NPWS | Management | N |  |  |
| HD7313 | Notification by NPWS of all Margaritifera monitoring transect locations to other monitoring agencies, to avoid any accidental damage by them | Protect | NPWS | Management | Y |  |  |
| HD7314 | Review water status, risks and pressures in catchments associated with M. margaritifera populations, in order to assist NPWS in identifying the more readily recoverable populations, and in identifying potential translocation rivers for M. durrovensis | Protect/Restore | Local Authorities / NPWS | Management | Y |  |  |
| HD7315 | On-going identification by NPWS of M. margaritifera populations for which there is evidence that existing pressures are such that populations may be recoverable, with a view to applying appropriate conservation measures to them | Protect/Restore | NPWS | Monitoring | Y |  |  |
| HD7316 | Investigative monitoring of river water quality parameters in association with NPWS rapid assessment monitoring | Protect/Restore | EPA/Local Authorities | Monitoring | Y |  |  |
| HD7317 | Review controls on use and disposal of pesticides including sheep-dip | Protect/Restore | EPA | Management | N |  |  |
| HD7318 | Notification and circulation of the Margaritifera species action plan when this has been completed by NPWS | Protect/Restore | NPWS | Management | N |  |  |
| Habitats Directive Measure | Action | Objective of measure | Responsible Agency | Type | GIS Locatable | Appropriate Assessment Required | High Status Sites Measure |
|  | Measures for Vertigo species |  |  |  |  |  |  |
| HD7321 | Assessment of known populations of Vertigo species by NPWS is on-going; it is recommended that assessments are updated annually to 2010 to inform Programmes of Measures for these species. | Protect/Restore | NPWS | Monitoring | Y |  |  |
| HD7322 | Map the locations of Vertigo populations in GIS, identify their relevant groundwater body(s) and delineate the zone of contribution, notify and circulate database to the relevant implementing agencies | Protect/Restore | NPWS | Data | Y |  |  |
| HD7323 | Assess requirements for remedial measures and implement | Protect/Restore | NPWS/Local Authorities | Management | N | Y |  |
| HD7324 | Review controls on use and disposal of pesticides including sheep-dip |  | EPA | Management |  |  |  |
|  | Avoidance of adverse impacts |  |  |  |  |  |  |
|  | Prevention of adverse impacts through planning, licensing and grant- aiding procedures |  |  |  |  |  |  |
| HD7411 | Use guidance on Article 6 assessment for Plans and Projects (Section 6 of Guidance Document) | Protect | Local Authorities/ An Bord Pleanala | Management | N | Y | HS 2 |
| HD7412 | Use sensitivities of water dependent habitats and species to water quality and hydrological pressures as set out in Sections 3 and 5 to guide decision making | Protect | Local Authorities / An Bord Pleanala / EPA | Management | N | Y | HS 3 |
| HD7413 | Consult with NPWS | Protect | All | Management | N | Y | HS 4 |
|  | Prevention of adverse impacts through development and implementation of Codes of Practice |  |  |  |  |  |  |
| HD742 | Develop a Code of Good Practice for construction activity in and adjoining watercourses. | Protect | EPA / NPWS / Fisheries Boards | Management | N |  | HS 5 |
| HD743 | Develop a code of good farming practice for the management of riparian zones, and the management of ditches and field drainage systems, with the aim of reducing siltation impacts on watercourses | Protect | EPA/DAFF | Management | N |  | HS 6 |
| Habitats Directive Measure | Action | Objective of measure | Responsible Agency | Type | GIS Locatable | Appropriate Assessment Required | High Status Sites Measure |
|  | Administrative - database measures |  |  |  |  |  |  |
| HD7511 | Update the Register of Protected Areas in respect of SACs designated under the Habitats Directive for Annex 1 listed water dependent habitats and Annex 2 species. The relevant field in the MapInfo SAC_Water_Dependency database is WD_H_S. This is a logical field (true/false), to be queried with the expression WD_H_S = “T” | Protect | EPA | Data | Y |  | HS 7 |
| HD7512 | The SAC Water Dependency database must be included in the EPA ENVision Online Mapviewer WFD Protected Areas | Protect | EPA | Data | Y |  | HS 8 |
| HD7513 | The SAC_Water_Dependency database should revert to NPWS for updating, with any updates notified annually to the EPA, which should then notify and disseminate updates to all agencies involved in the implementation of the Water Framework Directive | Protect | NPWS | Data | Y |  |  |
| HD7521 | Update Register of Protected Areas and EPA ENVision Online Mapviewer in respect of SPAs; additional sites have been designated since 2005 | Protect | EPA | Data | Y |  | HS 9 |
| HD7522 | Clarify which SPAs are designated for water dependent bird species | Protect | NPWS | Data | Y |  |  |
| HD7523 | NPWS should develop a second tier (shadow list) database of water dependent habitats and species occurrence in conservation sites designated under national legislation (NHAs, designated principally for bogs). It is also recommended that NPWS progresses with the formal designation of pNHAs to NHA status, accompanied by a boundary review to ensure that water dependent habitats and species are included, together with appropriate buffer zones | Protect | NPWS | Data /Management | Y |  | HS 10 |
| Habitats Directive Measure | Action | Objective of measure | Responsible Agency | Type | GIS Locatable | Appropriate Assessment Required | High Status Sites Measure |
|  | Database cross-referencing measures |  |  |  |  |  |  |
| HD7531 | NPWS habitat and species databases should include fields cross-referencing individual habitat features and species distribution to EPA water body codes for river sections, lakes, transitional waters, coastal waters, and ground waters | Protect/Restore | NPWS | Data | Y |  | HS 11 |
| HD7532 | EPA databases on water body status should include fields cross-referencing individual features to SAC Site Code, SPA Site Code and NPWS Site Name, where the features occurs within the boundaries of an SAC or SPA. Fields should also be included to cross-reference lakes to Habitats Directive Annex 1 listed lake habitats and turloughs, and updated as more habitat information becomes available from NPWS | Protect/Restore | EPA | Data | Y |  | HS 12 |
| HD7533 | The National Abstractions Register should include fields cross-referencing abstractions to the Site Code and Site Name of SACs and SPAs they are located in | Protect/Restore | EPA | Data | Y |  | HS 13 |
| HD7541 | It is recommended that a central Register of relevant databases is established, to include reference to all Water Framework Directive – relevant databases developed and maintained by all relevant agencies including NPWS, EPA, GSI, Central and Regional Fisheries Boards, Marine Institute, Forest Service, Local Authorities | Protect/Restore | INSPIRE/ DoEHLG/All | Data | N |  | HS 14 |
| HD7542 | All agencies holding relevant databases should establish procedures for regular update and notification of updates, and establish procedures for providing these data in GIS together with the associated metadata. Guidance on how data are to be used may also be necessary | Protect/Restore | All | Data | N |  | HS 15 |
| Habitats Directive Measure | Action | Objective of measure | Responsible Agency | Type | GIS Locatable | Appropriate Assessment Required | High Status Sites Measure |
| HD7543 | Agencies displaying information (via on-line map viewers) that is provided and up-dated by other agencies, should establish procedures for updating this information, and an on-line means of identifying which version is currently displayed | Protect/Restore | All | Data | N |  | HS 16 |
|  | Measures to encourage appropriate land uses within protected areas for water dependent habitats and species |  |  |  |  |  |  |
| HD761 | A review of Notifiable Activities by NPWS is in progress. It is recommended that these include review of the findings of all NPWS and other relevant monitoring programmes, e.g. Commonage Framework Plan monitoring. | Protect/Restore | NPWS | Management | N |  | HS 17 |
| HD762 | Administrative measures relating to the implementation of approved farm plans are recommended, to ensure that NPWS objectives are fully reflected in the plans. | Protect/Restore | NPWS | Management | N |  | HS 18 |
| HD763 | Supports for organic farming within Protected Areas are recommended | Protect/Restore | NPWS/DAFF | Management | N |  | HS 19 |
| HD764 | Nutrient and silt loading issues should be addressed with regard to the ecological requirements of individual Annex listed habitats and species present in individual Protected Areas, with provision for site-specific variation in soils and underlying geology, slope, livestock type, and seasonal restrictions on grazing and/or mowing regimes | Protect/Restore | NPWS | Management | Y |  | HS 20 |
| HD765 | Ex-situ issues and associated measures identified by NPWS as required to assist in returning individual Natura 2000 sites to favourable conservation status should be notified to the EPA and the relevant Local Authority | Protect/Restore | NPWS | Management | Y |  | HS 21 |
| Habitats Directive Measure | Action | Objective of measure | Responsible Agency | Type | GIS Locatable | Appropriate Assessment Required | High Status Sites Measure |
|  | Additional measures for High Status Sites |  |  |  |  |  |  |
| HD771 | Review sites for habitats and species which are at favourable conservation status under the Habitats Directive, and which are not covered by a conservation designation currently, apply an appropriate conservation designation under EU and/or national legislation | Protect | NPWS | Management | Y |  | HS 22 |
| HD772 | Ensure that all water bodies under consideration for listing as High Status Sites under the Water Framework Directive are at favourable conservation status for any Habitats Directive Annex 1 water dependent habitats and/or Annex 2 species known to be associated with them | Protect/Restore | EPA/NPWS | Management | Y |  | HS 23 |
| HD773 | Identify water bodies at high status, water bodies with WFD objective of restoring to high status catchment areas of monitoring sites at high status ((Q5 and Q4.5) in Regional, County and Local Development Plans | Protect/Restore | Local Authorities | Management | Y |  | HS24 |
| HD774 | Ensure that the appropriate high environmental quality standards are applied to all water bodies that would qualify for High Status under the Water Framework Directive, but where status has been down-graded to Moderate because an Annex 1 water dependent habitat or Annex 2 water dependent species is not at favourable conservation status under the Habitats Directive. | Restore | Local Authorities | Management | Y |  | HS 25 |
| HD775 | Ensure that the appropriate high environmental quality standards, as set out in the Draft Environmental Quality Objectives Regulations are applied to all high status water bodies and catchment areas of high status monitoring sites (Biological Quality rating Q4 and Q4.5). | Protect/Restore | Local Authorities | Management | Y |  | HS26 |
| Habitats Directive Measure | Action | Objective of measure | Responsible Agency | Type | GIS Locatable | Appropriate Assessment Required | High Status Sites Measure |
| HD776 | Prohibit point and diffuse source discharges liable to cause water pollution except where such discharges are subject to prior authorisation or general binding rules. Discharge authorisations must lay down emission limits that aim to achieve the environmental objectives/quality standards specified in the draft regulations | Protect | Local Authorities | Management |  |  | HS 27 |

## APPENDIX 13.   Draft European Communities Environmental Objectives (Surface Waters) Regulations 2008

SCHEDULE 5, Criteria for calculating surface water ecological status and ecological potential and  SCHEDULE 6: The environmental quality standards for priority substances and certain other pollutants to apply for the purpose of assigning chemical status

Table 8 Biological quality elements

RIVERS (All types)

| Biological quality element | Classification system | Ecological quality ratio |  |
| --- | --- | --- | --- |
|  |  | High – good boundary | Good – moderate boundary |
| Benthic invertebrate fauna | Quality rating system (Q -value) | 0.85 | 0.75 |
| Phytobenthos | Trophic diatom index (TDI) | 0.93 | 0.78 |

LAKES

| Biological quality element | Classification system | Lake type(1) | Ecological quality ratio |  | Chlorophyll a (µg/l) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | High-good | Good -moderate | High-good | Good - moderate |
| Phytoplankton | Phytoplankton biomass(2) (Chlorophyll a) | Lake type 4 | 0.50 | 0.33 | 6 | 9 |
|  |  | Lake types 7, 8, 11 and 12 | 0.55 | 0.32 | 5.8 | 10 |
| Macrophytes | Lake macrophytes | All types | 0.90 | 0.68 | N/A |  |

- Type 4: Low alkalinity (<20 mg/l CaCO3), deep (>4m) and large (>50 ha)

Type 7: Moderate alkalinity (20-100 mg/l CaCO3), deep (>4 m) and small (<50 ha)

Type 8: Moderate alkalinity (20-100 mg/l CaCO3), deep (>4 m) and large (>50 ha)

Type 11: High alkalinity (>100 mg/l CaCO3), deep (>4 m) and small (<50 ha)

Type 12: High alkalinity (>100 mg/l CaCO3), deep (>4 m) and large (>50 ha)                                        (2) Growing season (March to October) mean value

COASTAL WATERS (All types with the exception of coastal water lagoons)

| Biological quality element | Classification system | Ecological quality ratio |  | High-good boundary | Good-moderate boundary |
| --- | --- | --- | --- | --- | --- |
| Phytoplankton | Phytoplankton biomass (Chlorophyll) | High-good | Good -moderate | Chlorophyll a (µg/l)(1) |  |
|  |  | 0.66 | 0.33 | 2.5 (median value) and 5.0 (90 percentile value)(2) 5.0 (median value) and 10.0 (90 percentile value)(3) | 5.0 (median value) and 10.0 (90 percentile value)(2) 10.0 (median value) and 20 (90 percentile value)(3) |
|  | Phytoplankton composition | 0.84 | 0.43 | Percentage of single taxa counts above thresholds |  |
|  |  |  |  | 20 | 39 |

- Growing season March to September

- Cold acetone extraction method

- Hot methanol extraction method

COASTAL AND TRANSITIONAL WATERS (All types with the exception of transitional and coastal water lagoons)

| Biological quality element | Classification system | Ecological quality ratio |  |
| --- | --- | --- | --- |
|  |  | High – good boundary | Good – moderate boundary |
| Macroalgae | Rocky shore reduced species list multimetric system | 0.80 | 0.60 |
|  | Opportunistic macroalgae multimetric system | 0.80 | 0.60 |

# Table 9

Physico-chemical conditions supporting the biological elements

PART A: General conditions

THERMAL CONDITIONS

| Thermal conditions | River water body | Lake water body | Transitional water body | Coastal water body |
| --- | --- | --- | --- | --- |
| Temperature | <1.5˚C rise in ambient temperature downstream of a point of discharge |  |  |  |

OXYGENATION CONDITIONS (BIOCHEMICAL OXYGEN DEMAND)

| Oxygenation conditions | River water body | Lake water body | Transitional water body | Coastal water body |
| --- | --- | --- | --- | --- |
| Biochemical Oxygen Demand (BOD) (mg O2/l) | High status ≤1.3 (mean) or ≤2.2 (95%ile) Good status ≤1.5 (mean1) or ≤2.6 (95%ile) | N/A | ≤4.0 mg/l (95%ile) | N/A |

OXYGENATION CONDITIONS CONTINUED (DISSOLVED OXYGEN)

| Oxygenation conditions | River water body | Lake water body | Transitional water body (Summer) | Coastal water body (Summer) |
| --- | --- | --- | --- | --- |
| Dissolved oxygen lower limit | 95%ile >80% saturation |  | (0 psu1) 95%ile >70% saturation (35 psu) 95%ile >80% saturation | (35 psu) 95%ile > 80% saturation |
| Dissolved oxygen upper limit | 95%ile <120% Saturation |  | (0 psu) 95%ile <130% saturation (35 psu) 95%ile <120% saturation | (35 psu) 95%ile <120% saturation |

(1) psu: The Practical Salinity Unit defines salinity in terms of a conductivity ratio of a sample to that of a solution of 32.4356 g of KCL at 15oC in a I kg solution. A sample of seawater at 15oC with a conductivity equal to this KCL solution has a salinity of exactly 35 practical salinity units (psu).

ACIDIFICATION STATUS

| Acidification status | River water body | Lake water body | Transitional water body | Coastal water body |
| --- | --- | --- | --- | --- |
| pH (Individual values) | Soft) Water 4.5< pH < 9.0 HardWater 6.0< pH < 9.0 |  | N/A | N/A |

NUTRIENT CONDITIONS

| Nutrient conditions | River water body | Lake | Transitional water body | Coastal water body |
| --- | --- | --- | --- | --- |
| Total Ammonia (mg N/l) | High status ≤0.040 (mean1) or ≤0.090 (95%ile) Good status ≤0.065 (mean1) or ≤0.140 (95%ile) |  | N/A | N/A |
| Dissolved Inorganic Nitrogen (mg N/l) |  |  | N/A | High status: ≤ 12μM Good status: ≤18μM1 |
| Molybdate Reactive Phosphorus (MRP) (mg P/l) | High status ≤0.025 (mean1) or ≤0.045 (95%ile) Good status ≤0.035 (mean1) or ≤0.075 (95%ile) |  | (0-17 psu) ≤ 0.060 (median) (35psu) ≤ 0.040 (median) | - |

# Table 10

Physico-chemical conditions supporting the biological elements

PART B: Specific pollutants

AA: annual average(1)

MAC: maximum allowable concentration

Unit: [µg/l]

| Name of substance | Environmental quality standard (EQS)(2) () |  |  |  |
| --- | --- | --- | --- | --- |
|  | AA-EQS(5) Inland surface waters | AA-EQS Other surface waters | MAC-EQS(6) Inland surface waters | MAC-EQS Other surface waters |
| Arsenic | 25 | 20 | - | - |
| Chromium III | 4.7 | - | 32 | - |
| Chromium VI | 3.4 | 0.6 | - | 32 |
| Copper(3) | 5 or 30 | 5 | - | - |
| Cyanide | 10 | 10 | - | - |
| Diazinon | 0.01 | 0.01 | 0.02 | 0.26 |
| Dimethoate | 0.8 | 0.8 | 4 | 4 |
| Fluoride | 500 | 1,500 | - | - |
| Glyphosate | 60 | - | - | - |
| Linuron | 0.7 | 0.7 | 0.7 | 0.7 |
| Mancozeb | 2 | 2 | 7.3 | 7.3 |
| Monochlorobenzene | 1.5 | 25 | - | - |
| Phenol | 8 | 8 | 46 | 46 |
| Toluene | 10 | 10 | - | - |
| Xylenes | 10 | 10 | - | - |
| Zinc(4) | 8 or 50 or 100 | 40 | - | - |

      - The calculation of the arithmetic mean and the analytical method used must be in accordance with the proposed Directive adopting technical specifications for chemical monitoring and quality of analytical results in accordance with Directive 2000/60/EC of the European Parliament and of the Council, including how to apply an EQS where there is no appropriate analytical method meeting the minimum performance criteria.

      - The values for all substances, except Chromium III and Chromium VI, are for total concentrations. Values for chromium are for dissolved metal i.e. after filtration through a 0.45 micron filter. In designing monitoring programmes, it was recommended that total chromium be determined.  Where the result for total chromium is less than the EQS for Cr VI, no further investigation is deemed necessary.  Where the total Chromium level is above the EQS, an assessment should be made of the potential discharges of Cr VI into the waterbody.  Where there is a risk of Cr VI contamination, speciation studies should be included in the monitoring programme

      - The values for all metals, except Chromium VI, are as added values to background concentrations. In the case of Copper the value 5 applies where the water hardness measured in mg/l CaCO3 is less than or equal to 100; the value 30 applies where the water hardness exceeds 100 mg/l CaCO3

      - In the case of Zinc, the standard shall be 8 μg/l for water hardness with annual average values less than or equal to 10 mg/l CaCO3, 50 μg/l for water hardness greater than 10 mg/l CaCO3 and less than or equal to 100 mg/l CaCO3 and 100 μg/l elsewhere

      - AA-EQS means that for each representative monitoring point within the waterbody, the arithmetic mean of the concentrations measured over a twelve month monitoring period does not exceed the standard

      - MAC-EQS means that for each representative monitoring point within the waterbody no measured concentration exceeds the standard

SCHEDULE 6

# Table 11

The environmental quality standards for priority substances and certain other pollutants to apply for the purpose of assigning chemical status

With the exception of cadmium, lead, mercury and nickel (hereinafter "metals") the EQS values in tables 11 and 12 are expressed as total concentrations in the whole water sample. In the case of metals the EQS refers to the dissolved concentration, i.e. the dissolved phase of a water sample obtained by filtration through a 0.45 µm filter or any equivalent pre-treatment

- Priority Substances

AA: annual average;

MAC: maximum allowable concentration.

Unit: [µg/l]

| (1) | (2) | (3) | (4) | (5) | (6) | (7) |
| --- | --- | --- | --- | --- | --- | --- |
| N° | Name of substance | Chemical Abstracts Service number. | AA-EQS Inland surface waters | AA-EQS2 Other surface waters | MAC-EQS, Inland surface waters3 | MAC-EQS4 Other surface waters |
| (1) | Alachlor | 15972-60-8 | 0.3 | 0.3 | 0.7 | 0.7 |
| (2) | Atrazine | 1912-24-9 | 0.6 | 0.6 | 2.0 | 2.0 |
| (3) | Benzene | 71-43-2 | 10 | 8 | 50 | 50 |
| (4) | Carbon-tetrachloride | 56-23-5 | 12 | 12 | not applicable | not applicable |
| (5) | Chlorfenvinphos | 470-90-6 | 0.1 | 0.1 | 0.3 | 0.3 |
| (6) | Chlorpyrifos (Chlorpyrifos-ethyl) | 2921-88-2 | 0.03 | 0.03 | 0.1 | 0.1 |
| (7a) | Cyclodiene pesticides: Aldrin5 Dieldrin5 Endrin5 Isodrin5 | 309-00-2 60-57-1 72-20-8 465-73-6 | Σ=0.01 | Σ=0.005 | not applicable | not applicable |
| (7b) | DDT total5, | not applicable | 0.025 | 0.025 | not applicable | not applicable |
|  | para-para-DDT5 | 50-29-3 | 0.01 | 0.01 | not applicable | not applicable |
| (8) | 1,2-Dichloroethane | 107-06-2 | 10 | 10 | not applicable | not applicable |
| (9) | Dichloromethane | 75-09-2 | 20 | 20 | not applicable | not applicable |
| (10) | Di(2-ethylhexyl)-phthalate (DEHP) | 117-81-7 | 1.3 | 1.3 | not applicable | not applicable |
| (11) | Diuron | 330-54-1 | 0.2 | 0.2 | 1.8 | 1.8 |
| (12) | Fluoranthene | 206-44-0 | 0.1 | 0.1 | 1 | 1 |
| (13) | Isoproturon | 34123-59-6 | 0.3 | 0.3 | 1.0 | 1.0 |
| (14) | Lead and its compounds | 7439-92-1 | 7.2 | 7.2 | not applicable | not applicable |
| (15) | Naphthalene | 91-20-3 | 2.4 | 1.2 | not applicable | not applicable |
| (16) | Nickel and its compounds | 7440-02-0 | 20 | 20 | not applicable | not applicable |
| (17) | Octylphenol ((4-(1,1’,3,3’-tetramethylbutyl)-phenol)) | 140-66-9 | 0.1 | 0.01 | not applicable | not applicable |
| (18) | Pentachloro-phenol | 87-86-5 | 0.4 | 0.4 | 1 | 1 |
| (19) | Simazine | 122-34-9 | 1 | 1 | 4 | 4 |
| (20a) | Tetrachloro-ethylene5 | 127-18-4 | 10 | 10 | not applicable | not applicable |
| (20b) | Trichloro-ethylene5 | 79-01-6 | 10 | 10 | not applicable | not applicable |
| (21) | Trichloro-benzenes | 12002-48-1 | 0.4 | 0.4 | not applicable | not applicable |
| (22) | Trichloro-methane | 67-66-3 | 2.5 | 2.5 | not applicable | not applicable |
| (23) | Trifluralin | 1582-09-8 | 0.03 | 0.03 | not applicable | not applicable |

# Table 12

The environmental quality standards for priority hazardous substances to apply for the purpose of assigning chemical status

Priority Hazardous Substances

AA: annual average;

MAC: maximum allowable concentration.

Unit: [µg/l]

| (1) | (2) | (3) | (4) | (5) | (6) | (7) |
| --- | --- | --- | --- | --- | --- | --- |
| N° | Name of substance | Chemical Abstracts Service number. | AA-EQS Inland surface waters | AA-EQS2 Other surface waters | MAC-EQS, Inland surface waters3 | MAC-EQS4 Other surface waters |
| (1) | Anthracene | 120-12-7 | 0.1 | 0.1 | 0.4 | 0.4 |
| (2) | Brominated diphenylether | 32534-81-9 | 0.0005 | 0.0002 | not applicable | not applicable |
| (3) | Cadmium and its compounds (depending on water hardness classes) | 7440-43-9 | ≤ 0.08 (Class 1) 0.08 (Class 2) 0.09 (Class 3) 0.15 (Class 4) 0.25 (Class 5) | 0.2 | ≤ 0.45 (Class 1) 0.45 (Class 2) 0.6 (Class 3) 0.9 (Class 4) 1.5 (Class 5) |  |
| (4) | C10-13 Chloroalkanes | 85535-84-8 | 0.4 | 0.4 | 1.4 | 1.4 |
| (5) | Endosulfan | 115-29-7 | 0.005 | 0.0005 | 0.01 | 0.004 |
| (6) | Hexachloro-benzene | 118-74-1 | 0.01 | 0.01 | 0.05 | 0.05 |
| (7) | Hexachloro-butadiene | 87-68-3 | 0.1 | 0.1 | 0.6 | 0.6 |
| (8) | Hexachloro-cyclohexane | 608-73-1 | 0.02 | 0.002 | 0.04 | 0.02 |
| (9) | Mercury and its compounds | 7439-97-6 | 0.05 | 0.05 | 0.07 | 0.07 |
| (10) | Nonylphenol (4-Nonylphenol) | 104-40-5 | 0.3 | 0.3 | 2.0 | 2.0 |
| (11) | Pentachloro-benzene | 608-93-5 | 0.007 | 0.0007 | not applicable | not applicable |
| (12) | Polyaromatic hydrocarbons (PAH)7 | not applicable | not applicable | not applicable | not applicable | not applicable |
|  | Benzo(a)pyrene | 50-32-8 | 0.05 | 0.05 | 0.1 | 0.1 |
|  | Benzo(b)fluor-anthene | 205-99-2 | Σ=0.03 | Σ=0.03 | not applicable | not applicable |
|  | Benzo(k)fluor-anthene | 207-08-9 |  |  |  |  |
|  | Benzo(g,h,i)-perylene | 191-24-2 | Σ=0.002 | Σ=0.002 | not applicable | not applicable |
|  | Indeno(1,2,3-cd)-pyrene | 193-39-5 |  |  |  |  |
| (13) | Tributyltin compounds (Tributhyltin-cation) | 36643-28-4 | 0.0002 | 0.0002 | 0.0015 | 0.0015 |

Section 5.3.1., Managing Natura 2000 sites: The provisions of Article 6 of the ‘Habitats’ Directive 92/43/EEC. ISBN 92-828-9048-1 © European Communities, 2000. Available on http://ec.europa.eu/environment/nature/natura2000/management/guidance_en.htm

Population named after river of highest stream-order that contains mussels.

The calculation of the arithmetic mean and the analytical method used must be in accordance with the proposed Directive adopting technical specifications for chemical monitoring and quality of analytical results in accordance with Directive 2000/60/EC of the European Parliament and of the Council, including how to apply an EQS where there is no appropriate analytical method meeting the minimum performance criteria.

Water hardness ≤ 100 mg/l CaCO3

Water hardness > 100 mg/l CaCO3

Salinity related

The calculation of the arithmetic mean and the analytical method used must be in accordance with the proposed Directive adopting technical specifications for chemical monitoring and quality of analytical results in accordance with Directive 2000/60/EC of the European Parliament and of the Council, including how to apply an EQS where there is no appropriate analytical method meeting the minimum performance criteria.

This parameter is the Environmental Quality Standard expressed as an annual average value (EQS-AA). Unless otherwise specified, it applies to the total concentration of all isomers.

Inland surface waters encompass rivers and lakes and related artificial or heavily modified water bodies.

This parameter is the Environmental Quality Standard expressed as a maximum allowable concentration (MAC-EQS). Where the MAC-EQS are marked as "not applicable", the AA-EQS values are considered protective against short-term pollution peaks in continuous discharges since they are significantly lower than the values derived on the basis of acute toxicity.

This substance is not a priority substance but one of the other pollutants for which the EQS are identical to those laid down in community legislation that applied prior to the proposed Directive on environmental quality standards in the field of water policy.

DDT total comprises the sum of the isomers 1,1,1-trichloro-2,2 bis (p-chlorophenyl) ethane (CAS number 50-29-3; EU number 200-024-3); 1,1,1-trichloro-2 (o-chlorophenyl)-2-(p-chlorophenyl) ethane (CAS number 789-02-6; EU Number 212-332-5); 1,1-dichloro-2,2 bis (p-chlorophenyl) ethylene (CAS number 72-55-9; EU Number 200-784-6); and 1,1-dichloro-2,2 bis (p-chlorophenyl) ethane (CAS number 72-54-8; EU Number 200-783-0).

The calculation of the arithmetic mean and the analytical method used must be in accordance with the proposed Directive adopting technical specifications for chemical monitoring and quality of analytical results in accordance with Directive 2000/60/EC of the European Parliament and of the Council, including how to apply an EQS where there is no appropriate analytical method meeting the minimum performance criteria.

This parameter is the Environmental Quality Standard expressed as an annual average value (EQS-AA). Unless otherwise specified, it applies to the total concentration of all isomers.

Inland surface waters encompass rivers and lakes and related artificial or heavily modified water bodies.

This parameter is the Environmental Quality Standard expressed as a maximum allowable concentration (EQS-MAC). Where the MAC-EQS are marked as "not applicable", the AA-EQS values are considered protective against short-term pollution peaks in continuous discharges since they are significantly lower than the values derived on the basis of acute toxicity.

For the group of priority substances covered by brominated diphenylethers listed in Decision 2455/2001/EC, an EQS is established only for congener numbers 28, 47, 99, 100, 153 and 154.

For Cadmium and its compounds the EQS values vary dependent upon the hardness of the water as specified in five class categories (Class 1: <40 mg CaCO3/l, Class 2: 40 to <50 mg CaCO3/l, Class 3: 50 to <100 mg CaCO3/l, Class 4: 100 to <200 mg CaCO3/l and Class 5: ≥200 mg CaCO3/l).